## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg.

Subject: Principles of Airconditioning(5047) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of psychrometric charts, steam tables and Mollier’s

charts is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following psychrometric properties: i) Dew point temperature ii) Relative humidity iii) Wet bulb temperature. |  |
| b) | Define the term ‘by-pass factor’ as applied to cooling or heating coils and write expression for it. |  |
| c) | Explain the terms: i) Ventilation load ii) Effective sensible heat factor. |  |
| d) | State three major differences between central and unitary air conditioning systems. |  |
| e) | State three important considerations in the selection of air filters for airconditioners. |  |
| f) | Define the terms: i) Throw ii) Drop iii) Spread, as applied to airconditioners. |  |
| g) | Explain the meaning of “Economic Insulation Thickness”. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a laboratory test, a psychrometer recorded 36ºC DBT and 30ºC WBT. Calculate the following: (i) Vapour pressure (ii) Relative humidity (ii) Specific humidity (iv) Degree of saturation (v) Dew point temperature (vi) Enthalpy of mixture. |  |
| b) | Outside air at 31ºC DBT and 18.5ºC WBT enters a cooling coil at the rate of 40m3/min. The effective surface temperature of the cooling coil is 4.5ºC and its cooling is 12.5 KW of refrigeration. Find - DBT and WBT of air leaving the coil - Enthalpy of air leaving the coil - By-pass factor of the coil |  |
| c) | Explain the working of year-round air conditioning system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following data relates to the office air conditioning plant having maximum seating capacity of 25 occupants: Outside design conditions : 34ºC DBT, 28ºC WBT Inside design conditions : 24ºC DBT, 50% RH Solar heat gain : 9120W Latent heat gain per occupant : 105W Sensible heat gain per occupant : 90W Lighting load : 2300W Sensible heat load from other : 11630W sources Infiltration load :14 m3/min Assume 40% fresh air and 60% of recirculated air passing through the evaporator coil and the by-pass factor of 0.15, find the dew point temperature of the coil and capacity of the plant. |  |
| b) | Discuss the different types of heat loads which have to be taken into account in order to estimate the total load of a large restaurant for summer air conditioning. |  |
| c) | Explain the working of a district air condition system. | …..2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the special features of air conditioning system used in hospitals. |  |
| b) | Explain the construction and working of electric filters. |  |
| c) | Explain the construction and working of air washer humidifier. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the considerations in the selection and location of supply air outlets? Name the different types of supply air outlets and state their applications. |  |
| b) | What are the desirable properties of ideal insulating materials? State the different types of insulating materials applicable for i) Cold surface ii) Hot surface. |  |
| c) | Explain the working of Two-position pneumatic control system. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What is sling psychrometer? Explain with a neat sketch. |  |
| b) | Illustrate mixing process on a psychrometric chart and derive an expression for enthalpy of mixture when mass and enthalpy of two streams before mixing is known. |  |
| c) | Explain ‘Equal friction method’ of duct design. |  |
| d) | Explain any one type of humidity control mechanism used in air conditioning. |  |

***********