## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction To Thermodynamics(SH404)/(4334) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam tables is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between open system and closed system. Give one example for each system. |  |
| b) | Draw P-V and T-S diagram for Isothermal compression process. |  |
| c) | Define following terms as referred to Internal Combustion Engines: - Swept volume ii) Volumetric efficiency |  |
| d) | Define following terms as referred to steam: - Dryness fraction ii) Degree of super heat |  |
| e) | Differentiate between parallel flow heat exchanger and counter flow heat exchanger. |  |
| f) | Define: i) Refrigeration ii) Tonne of Refrigeration (TR). |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain specific heat at constant pressure and specific heat at constant volume. Why specific heat at constant pressure is always more than specific heat at constant volume? |  |
| b) | i) 5m3 of air is compressed at constant temperature to a final volume of 0.5m3. If the initial pressure is 2 bar, what will be the final pressure of the air in KPa? |  |
|  | ii) The pressure of a gas supplied to an engine is 1.3 bar. Determine the volume of 1.8 Kg of this gas if its temperature is 93ºC. Take R= 0.722 KJ/KgK |  |
| c) | What are the functions of lubricating systems? Explain in brief any one method of lubrication system in Internal Combustion Engine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Four Kg of gas occupying 0.5m3 is heated from 60ºC to 115ºC at constant pressure of 774 KPa. Find: i) Final volume of gas in m3 ii) Gas constant. |  |
| b) | A certain quantity of air in a closed vessel of fixed volume of 0.15 m3 exerts a pressure of 1 MPa at 500K. If the vessel is cooled so that the pressure falls to 350 KPa, determine: i) the final temperature of air in Kelvin (K) ii) change in internal energy iii) the heat removed. |  |
|  | Take R =0.287 KJ/KgK, Cv = 0.7165 KJ/KgK |  |
| c) | Two Kg of air at a pressure of 690 KPa occupy a volume of 0.3m3. This air is then expanded adiabatically to a volume of 1.5m3. Find: i) the final temperature of air ii) work done. |  |
|  | Take R = 0.287 KJ/KgK, CP = 1.0035 KJ/KgK and Cv = 0.7165 KJ/KgK |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of P-V diagram, explain Dual cycle. |  |
| b) | Explain with the help of neat sketch a Two-stroke cycle Diesel Engine. |  |
|  |  | …..2/- |
|  | -2- |  |
| c) | The following results were obtained during the test on a single-cylinder oil engine: |  |
|  | Duration of Test : 50 minutes |  |
|  | Fuel oil used : 2.633 Kg |  |
|  | Average Indicated power : 13.32 KW |  |
|  | Average Brake power : 10.48 KW |  |
|  | Calorific value of fuel : 44,400 KJ/Kg |  |
|  | Calculate i) the Indicated thermal efficiency |  |
|  | ii) the Brake thermal efficiency of the engine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How much heat is required to convert 5 Kg of water at 45ºC into steam at pressure of 4 bar and 80 percent dry? |  |
|  | Take specific heat of water as 4.187 KJ/KgK |  |
| b) | Use the steam table to find enthalpy and specific volume of steam at 16 bar and 230ºC. Take specific heat at constant pressure for superheated steam as 2.1 KJ/KgK. |  |
| c) | A furnace wall is made of refractory bricks of 410 mm thick. The inner and outer surfaces of the wall have temperatures of 950ºC and 90ºC. Find the heat loss per sqm per hour. Assume K = 4.5 W/mK for refractory bricks. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the functions of following components of vapour compression refrigeration system: - Compressor ii) Condenser iii) Receiver |  |
| b) | Write a note on application of refrigeration on ships. |  |
| c) | A refrigeration system uses R–134a as refrigerant and works between temperature limits of -20ºC and 30ºC. If the refrigerant enters the compressor as dry saturated vapour and leaves the condenser in saturated liquid state, find the following: i) Carnot C.O.P ii) Theoratical C.O.P. |  |
|  | Use the following properties of R–134a |  |
|  | Temp ºC; Enthalpy KJ/Kg. Liquid hf; Latent hfg; Vapour hg. -20ºC; 173.82; 212.84; 386.66. 30ºC; 241.65; 173.29; 414.94. |  |
|  | Temperature at the end of compression = 42ºC, specific heat of refrigerant vapor = 1.044 KJ/KgK |  |

***********