## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC

Subject: Audio & Video Engineering(EN601) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief distortion & frequency response in respect of a microphone. |  |
| b) | Explain in brief the terms: picture frame, pixels & aspect ratio. |  |
| c) | Define hue, luminance & saturation |  |
| d) | What is interlace error? Explain for 16µs delay. |  |
| e) | Compare VCD & DVD. |  |
| f) | Differentiate between CCTV & cable TV. |  |
| g) | Explain the terms i) primary colours ii) complementary colours. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with neat diagram construction & working of crystal microphone. |  |
| b) | With the help of neat diagram explain the principle of working of a horn type loudspeaker. |  |
| c) | Write short note on baffles & enclosures. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with suitable sketches, how video signal is developed in a Vidicon camera. |  |
| b) | Sketch composite video signal waveform for at least three successive lines & indicate i) peak white level ii) blanking level iii) pedestal height iv) D.C level v) Picture details vi) Horizontal sync pulses. |  |
| c) | What is flicker? How interlaced scanning reduces flicker? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain various conditions for compatibility between monochrome & colour TV. |  |
| b) | With the help of neat block diagram explain the working of PAL TV receiver. |  |
| c) | With the help of neat diagram explain construction & working of Trinitron picture tube. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat block diagram explain the working principle of CCTV system. |  |
| b) | With the help of neat block diagram explain the principle of working of cable TV system. |  |
| c) | Sketch the details of horizontal blanking & sync pulse labelling the important regions. Why are front porch & back porch intervals provided before & after horizontal sync pulse? |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | VCD |  |
| b) | DVD |  |
| c) | Blu-ray disc player |  |

***********