## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Civil Engg./CT

Subject: Transportation Engineering-I(4006) [old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the merits and demerits of Airways. |  |
| b) | Why is it necessary to do soil stabilization during construction of roads? |  |
| c) | Distinguish between Prime coat and Seal coat. |  |
| d) | Explain the classification of bridges depending on alignment. |  |
| e) | State merits and demerits of concrete roads. |  |
| f) | Why is it necessary to construct tunnels in roadways & airways? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a National Highway in full cutting and mark all its component parts. |  |
| b) | What are the different types of curves used in roads? Explain each type in brief. |  |
| c) | Explain the construction procedure for W.B.M Roads. Also mentions the precautions to be taken during rolling. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various defects that occur in Bituminous roads? |  |
| b) | What are the two types of surface dressing done on bituminous roads? Explain each in brief. |  |
| c) | Explain Alternate Bay method of construction of concrete roads. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the basic principles of geometric design for hill roads with respect to sight distance and curves. |  |
| b) | Explain the term Arboriculture. Why and how is it done? |  |
| c) | Define i) Afflux ii) Economic span of bridge iii) Cut water and ease water. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat sketches explain Pile foundation and Caisson foundation for bridges. |  |
| b) | What is a Causeway? Explain in brief any two types of causeways. |  |
| c) | i) Write a note on special maintenance of bridges. ii) What are the various types of gauges used in Railways? |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Breakwater |  |
| b) | Alignment of bridges |  |
| c) | Bearings in bridges |  |
| d) | Super elevation |  |

***********