## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(ME614) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat sketch to show the following: - Wheel track ii) Wheel base |  |
| b) | Distinguish between timed and continuous injection, in gasoline fuel injection systems. |  |
| c) | What are the factors affecting the specific gravity of battery electrolyte? |  |
| d) | Distinguish between wheel alignment and wheel balancing. |  |
| e) | Distinguish between fluid coupling and torque convertor. |  |
| f) | Explain rolling of a vehicle. Name the part of the suspension system that checks the vehicle from rolling. |  |
| g) | List the possible causes and remedies for spongy brakes. |  |
| h) | Draw neat sketch of a ‘piston ring expander’ tool. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the power flow in a 4WD vehicle. |  |
| b) | List the various types of loads on a chassis frame. |  |
| c) | With the help of a neat sketch, explain the construction of a chassis frame. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of a single plate dry clutch. |  |
| b) | What is double declutching? Why is it done? Is it required to be done in a synchromesh gearbox? Why? |  |
| c) | With the help of a neat sketch, explain the working of an overdrive. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the use of stroboscope in spark ignition timing of a petrol engine. |  |
| b) | Draw a block diagram to show the Engine Management System of an MPFI engine. List the various sensors and state their location and function. |  |
| c) | With the help of a neat sketch, explain the working of an oil pressure warning light. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat flow diagram for the air-conditioning system of an automobile. List the types of compressors used and explain working of any one type. |  |
| b) | Discuss the use of R134a in car air conditioning. |  |
| c) | With the help of a neat sketch, explain the working of hydraulic brake system of an automobile. |  |
| d) | Draw the simplified wiring circuit diagram for the lighting system of an automobile |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of steering linkage for independent suspension vehicle. | …….2/- |
|  | -2- |  |
| b) | With the help of a neat sketch, explain the working of linkage type hydraulic power steering. |  |
| c) | With the help of a neat sketch, explain the working of McPherson strut type independent suspension. |  |
| d) | With the help of neat sketches, describe various types of pullers. |  |

***********