## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/COMP/EI

Subject:Basic Electrical Engg.(CS304)/Elements of Electrical Engg.(3001)

[Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain KVA rating of a transformer. |  |
| b) | State any three applications of a DC series motor. |  |
| c) | Explain how you can reverse the direction of rotation of a 415V, 10HP, 3-phase delta connected induction motor. |  |
| d) | Explain in brief system earthing. |  |
|  | (only for students of Revised scheme) |  |
| e) | For a sinusoidal AC voltage define the following: - RMS value ii) Form factor |  |
| f) | Explain in brief the concept of phase sequence of a three-phase supply. |  |
| g) | For a three phase induction motor with a star connected stator winding, state the equations to explain the relation between the following: - Line current and Phase current - Line voltage and phase voltage |  |
|  | (only for students of Old scheme) |  |
| e) | Explain the colour codes for identification of insulated conductors. |  |
| f) | Compare 3 phase 3 wire and 3 phase 4 wire distribution system. |  |
| g) | What are the advantages and disadvantages of cleat type wiring systems? Give its applications. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the principle of operation of a transformer. |  |
| b) | Explain in detail the various losses taking place in a transformer. |  |
| c) | With reference to a transformer, explain the following terms: i) Efficiency ii) Voltage regulation. |  |
| d) | With a neat diagram, explain the general construction of PVC insulated cable. (only for students of Old scheme) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain the functions of any three non-current carrying parts of dc motor. |  |
| b) | With neat sketches, explain how the direction of rotation of a DC shunt motor can be reversed. |  |
| c) | Explain back emf in a dc motor. What is its significance in the operation of a dc motor? |  |
| d) | Mention the advantages, disadvantages and applications of overhead and underground distribution system. (only for students of Old scheme) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain the principle of operation of an universal motor. |  |
| b) | State six points of differences between a squirrel cage and slip ring induction motor. | ……2/- |
|  | -2- |  |
| c) | Draw the torque-slip characteristics of a slip ring induction motor for two different rotor resistances. Also state any two applications of this motor. |  |
| d) | Compare PVC surface and concealed wiring system with respect to at least six points. (only for students of Old scheme) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain plate electrode. |  |
| b) | Explain any three I.E. rules relevant to earthing. |  |
| c) | Differentiate between HRC fuse and re-wirable fuse. (Any six points of differences) |  |
| d) | State any 6 important I.E. rules for domestic wiring system. (only for students of Old scheme) |  |
| Q.No.6. | Write notes on any two: | 2 x 6 = 12 |
| a) | ELCB |  |
| b) | Cartridge fuse and its ratings |  |
| c) | MCB |  |

***********