## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg./FT&EE

Subject: Elements of Civil Engineering(CS311)/(3003) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is meant by ranging? Differentiate direct ranging from indirect ranging. |  |
| b) | Define ‘Bearing of a line’. Show with sketch a line AB and indicate its Fore bearing and Back bearing. |  |
| c) | Define Back sight and Fore sight in levelling work. |  |
| d) | In respect of a Theodolite, define: Face left, Face right, Swinging and Transiting. |  |
| e) | Draw a neat proportionate sketch (section) of a spread footing for wall and indicate approximate dimensions. |  |
| f) | What is bearing capacity of soil? State its significance in building construction. |  |
| g) | Differentiate between Load bearing structures and Framed structures in buildings. (only for students of revised scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the aid of neat sketch method of measurement of distance between two points when there is a building as obstacle. |  |
| b) | i) Convert the following W.C.B. into Q.B. 13630, 21015, 30345. |  |
|  | ii) Find the included angles (Angles B, C and D) of an open traverse ABCDE when the observed bearing of the lines are as follows. AB 3015, BC 13050, CD 17500, DE 23145 The computation may be supported with sketches. |  |
| c) | Define in respect of leveling: i) Level line ii) Horizontal line iii) Bench Mark and T.B.M. iv) Line of collimation v) Change point vi) Focusing the eye piece. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following consecutive readings were taken with a dumpy level in a levelling work on a continuously sloping ground. 0.850, 1.105, 1.320, 1.645, 2.300, 0.900, 1.415, 1.880, 1.905, 0.655, 0.985, 1.225 Enter the readings neatly in a standard levelling format and compute the Reduced levels of various points. The first reading is taken on a point whose R.L. is to be assumed as 50.00 m. |  |
| b) | Using a theodolite, the vertical angles are observed towards the bottom and top of a tower and the angles are 630 and 1545 respectively. The sloping distance from the instrument station point to the bottom of the tower and is measured as 250.00m. Compute the height of the tower showing the heights and angles in a sketch. | …….2/- |
|  | -2- |  |
| c) | Define: i) Bearing capacity of soil ii) Safe Bearing capacity iii) Ultimate Bearing capacity iv) End bearing piles v) Friction piles vi) North light roof truss. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the construction of flooring suitable for workshop building. |  |
| b) | Write short notes on roof coverings and explain in brief the method of fixing A.C./ G.I. sheet roofing with sketches as needed. |  |
| c) | What are the main ingredients of cement concrete? Explain necessary precautions to be taken while mixing and transporting the concrete mixes. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain various curing methods used for concrete structures. |  |
| b) | Draw a neat plan (double line) of a washroom (toilet) of size 3.00m x 2.00m that includes an Indian W.C., wash basin, shower point, towel rail, door of size 0.80m x 2.10m, window/ventilator of 1.00m x 1.00m. Take scale of 1:20 and wall thickness of 20 cms. |  |
| c) | Draw a neat section of a wall from foundation to roof, for a ground plus one floor building and indicate plinth height, ceiling level, sill level, eaves level, ridge level, lintel level. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Acoustic insulation for an auditorium building |  |
| b) | Thermal insulation in buildings. |  |
| c) | Stepwise procedure to measure a horizontal angle accurately using a theodolite |  |
| d) | Importance of taking both fore bearing and back bearing of every line in a compass traverse and its use in detecting the local attraction |  |

***********