## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Architectural Engg.

Subject: Theory of Structures(AE405)/Structures-I(4537) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following: i) Mass ii) Weight iii) Force. |  |
| b) | Define water cement ratio and list the significance. |  |
| c) | Draw a neat sketch of load bearing wall footing. |  |
| d) | Enlist the causes of formation of structural cracks. |  |
| e) | State the difference between long column and short column. |  |
| f) | Enlist the different types of loads acting on structures. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the classification of system of forces. |  |
| b) | Outline the steps involved in analytical method for determination of resultant of co-planar force system. |  |
| c) | For a system of concurrent forces if the resultant is zero, find the magnitude and direction of force P. Refer fig.1. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Construct shear force and bending moment diagram for a cantilever shown in fig.2. |  |
| b) | Construct shear force and bending moment diagram for the simply supported beam shown in fig.3. |  |
| c) | What are sagging and hogging moments? Draw and explain: i) Cantilever beam ii) Fixed beam iii) An overhang beam. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the centre of gravity of the channel section whose flanges are 40mm x 10mm and web is 60mm x 10mm. The total height of channel is 80mm. |  |
| b) | Explain the detailed classification of structure, based on i) usage ii) load transfer. Also explain solid and skeleton structure. |  |
| c) | Explain approximate design guidelines for load bearing wall and wall footing. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the role of foundation movements and settlement in crack formation. |  |
| b) | Draw the neat sketch of: - Corbelling - Vaults - Arches |  |
| c) | Explain in detail, how cracks can be prevented by using i) proper architectural design ii) proper foundation design. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Law of parallelogram of forces |  |
| b) | Importance of architectural features and structural shapes in earthquakes zones |  |
| c) | Stresses formed due to bending in structure |  |
| d) | Grades of concrete and uses |  |
| e) | Working stress and limit state method of design |  |