## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/EI/MED/E&EE

Subject: Circuits & Networks(EN403)/(4136) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define voltage, mesh and active element. |  |
| b) | Explain in brief ideal voltage sources. |  |
| c) | Draw circuit diagram and explain Kirchhoff’s current law. |  |
| d) | Draw RC differentiator circuit and sketch input & output waveforms for sine and square wave inputs only. |  |
| e) | What is iterative impedance? |  |
| f) | Explain in brief characteristic impedance of a symmetrical network. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find equivalent resistance between terminals AB of fig.(1) using Delta/Star conversion. |  |
| b) | Find the current I in the circuit of fig.(2) using superposition theorem |  |
| c) | Using nodal analysis, find the current I in the circuit of fig.(3). |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find ammeter current in fig.(4) by using mesh analysis. |  |
| b) | Find current flowing through RL, using Thevenin’s theorem for the circuit in fig.(5). |  |
| c) | For the circuit shown in fig.(6), find the value of RL for maximum power transfer. Also, state the Maximum Power Transfer theorem. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following for RLC series resonance circuit: Resonance curve for different values of R, Half power bandwidth and Q-factor. |  |
| b) | An R-L-C series circuit is connected to 200V variable frequency supply. If R = 20Ω, L=20 mH, C=0.5µF, calculate the following: (i) resonant frequency (ii) Q-factor (iii) half power bandwidth (iv) maximum power dissipated at fo. |  |
| c) | A series circuit of R=10Ω, L=50mH and C=100µF is supplied with 200V, 50Hz. Find (i)Current (ii) impedance (iii) Voltage across R, L and C (iv) Power factor. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For a symmetrical – network ZSC = 75Ω, ZOC = 175Ω. Find Z1, Z2 and Zo. |  |
| b) | Find current I1 and I2 in the circuit of fig.(7). |  |
| c) | Draw symmetrical T-network and derive expressions for ZO, ZOC and ZSC. (only for students of revised scheme) |  |
| c) | A series RL circuit with R=30Ω and L=15mH has a constant voltage of 60V applied at t=0. Determine the current I, voltage across resistor R and voltage across inductor L. (only for students of old scheme) |  |
|  |  | ……..2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw symmetrical T and  attenuator circuits. Design a symmetrical T attenuator to give 25dB attenuation and having characteristics impedance of 600Ω. |  |
| b) | Draw and explain graphical representations of LPF, HPF, BPF, BRF. |  |
| c) | Determine the circuit elements of constant – K low pass filter composed of T-section to operate with RK=600Ω and fC=3KHZ. Draw diagram and graphical representation. (only for students of revised scheme) |  |
| c) | Explain the response of a RC circuit with a neat circuit diagram, necessary equations and also sketch its response. (only for students of old scheme) |  |