## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/EI/E/E&EE

Subject: Electronic Instrumentation(EN404) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Transducer. Draw the symbol of photodiode, phototransistor. |  |
| b) | List any three characteristics of an Ideal transducer. |  |
| c) | What is the objective of an Instrumentation system? |  |
| d) | List any three linear displacement measuring Transducers. |  |
| e) | What are filters? List types of filters. |  |
| f) | What is the need for signal conditioning? |  |
| g) | What are Plotters? List any 2 Plotters. |  |
| h) | List the application areas of SCADA. (any 3) |  |
| i) | Draw only block diagram of process control. |  |
| j) | How pressure is measured using diaphragm with strain gauge. |  |
| Q.No.2. | Answer any three following Questions: | 3 x 4 = 12 |
| a) | Draw a neat block diagram of a general Instrumentation system and explain the function of each block. |  |
| b) | Explain principle of working of a Capacitive Transducer using overlapping areas of a parallel plate capacitor. |  |
| c) | Explain construction and operation of shaft encoder. |  |
| d) | What is seeback effect? Explain Thermocouple and list materials used in its construction along with their temperature ranges. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain angular speed measurement using magnetic pick-up. |  |
| b) | Explain flow measurement using turbine meter. |  |
| c) | Using Inductive Transducers, how linear displacement measurement is done. Explain any one of it. |  |
| d) | Explain vibration measurement using Piezoelectric accelerometer. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are amplifiers? Explain a DC Amplifier. |  |
| b) | Explain a LED Digital Display. List its advantages. |  |
| c) | With the help of a neat circuit diagram explain Instrumentation Amplifier using op-amps. |  |
| d) | Explain any two types of filters. |  |
| Q.No.5. | With the help of a neat block diagram, explain any 2 of the following: i) Data Acquisition system ii) SCADA iii) Process control. Also list their respective application areas. | (12) |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Photo – voltaic cell |  |
| b) | Displacement measurement using Resistive method |  |
| c) | Level measurement using capacitive method |  |
| d) | Bridge circuits for strain gauge |  |
| e) | LDR |  |

***********