## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Piping Fabrication & Installation(FE611) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Refer data provided at the end of the paper.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give brief description of various types of Polyethylene pipes in use. |  |
| b) | Draw a simple Process-Flow-Diagram (PFD). |  |
| c) | Explain the working of a steam trap with the help of a neat diagram. |  |
| d) | Differentiate between Vertical down and Vertical up welding technique used in cross-country pipeline. |  |
| e) | For the pipeline shown in Fig.1(a) & (b) draw the developed view. |  |
| f) | Draw single line symbols for the following fittings: - Long bend radius elbow with screwed ends - Welded reducer - Flange connection |  |
| g) | Discuss the effect of changes in sizes of pipelines on the velocity of flow. |  |
| Q.No.2. | Design a pipe of ASTM A106 GB standard with following working condition: Temperature=250C; back pressure at end of pipeline=5 bar, length of pipeline=180m; equivalent length of all pipe fittings and valves=10m; flow rate = 28.39m3/hr. Other service conditions: Corrosion factor C=1.6mm for used material, Joint efficiency=0.8 and Y-factor is 0.4 for given temperature range. Select schedule, diameter and wall thickness using additional data given in the end. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the Front view, top view, side view and developed view of the portion of the pipeline whose Isometric view is shown in Fig.2. | (8) |
| b) | Make a Bill of Material of the pipeline shown in Fig.2,Q.3 (a). Neglect weight of fluid, pipes and fittings. | (4) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the steps involved in hydro testing of pipelines. |  |
| b) | Draw a sketch of section of an insulated pipeline and explain hot insulation process. |  |
| c) | Explain any one process of pipe manufacture in detail. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the developed view of hole to be made in a  100mm O.D. pipe to connect a  50mm pipe at an angle of 60 to the main axis. |  |
| b) | Write a note to explain important aspects of hot bending of pipes. |  |
| c) | What are the special considerations for design of steam pipelines? | …….2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Equipments orientation diagram with nozzle schedule |  |
| b) | The extrusion process of pipe manufacture |  |
| c) | Economic considerations for pipe selection |  |
| d) | Fabricated bends |  |
| e) | Special fittings and use of gaskets in pipelines |  |

Table 2: Pipe properties

| Wall thickness (inch) Pipe NB (inch) | 40 Sch | 80 Sch | 160 Sch | OD (inch) |
| --- | --- | --- | --- | --- |
| 1 | 0.133 | 0.179 | 0.250 | 1.315 |
| 1¼ | 0.140 | 0.191 | 0.261 | 1.660 |
| 1½ | 0.145 | 0.200 | 0.281 | 1.900 |
| 2 | 0.171 | 0.218 | 0.311 | 2.375 |
| 2½ | 0.203 | 0.276 | 0.375 | 2.875 |

Table 3: Allowable stresses of ASTM A106 Gr.B

| Temperature F | -20 to 650 | 700 | 750 | 775 |
| --- | --- | --- | --- | --- |
| Allowable stress ‘S’ psi | 15,000 | 14,300 | 12,900 | 11,850 |