## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg.

Subject: Power Plant Engineering(ME623)/(5037) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following: - Plant use factor ii) Demand factor iii) Plant capacity |  |
| b) | Draw a neat sketch of closed cycle gas turbine Power Plant. |  |
| c) | What factors are to be considered while selecting a site for thermal Power Plant? |  |
| d) | What are the different effects of contamination of water? |  |
| e) | How are hydal Power Plants classified? |  |
| f) | What is the necessity of cooling towers in Power Plants? |  |
| g) | Write a short note on radio active disposal. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The loads on a Power Plant with respect to time for 24 hours are given below: Time in Hours; 0-6; 6-8; 8-12; 12-14; 14-18; 18-22; 22-24. Load in MW; 40; 50; 60; 50; 70; 80; 40. - Draw the load curve and find out the load factor of the power station. - If the loads above 60MW are taken by a standby unit of 20MW capacity, find out the load factor of the standby unit. |  |
| b) | Draw a neat sketch of storage type hydal Power Plant and explain its operation. |  |
| c) | Draw the layout of medium capacity diesel Power Plant and explain it briefly. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How are gas turbine Power Plants classified? What are the advantages & disadvantages of using open cycle gas turbine Power Plant? |  |
| b) | What are the various steps involved in the preparation of coal? Name the equipments used in coal preparation. |  |
| c) | Explain with a neat sketch the general layout of ash handling plant. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the Pulverised coal handling system. |  |
| b) | With a neat sketch explain the working of steam Power Plant. |  |
| c) | Draw the general layout of water treatment plant and explain the importance of water treatment in steam Power Plant. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the forced draft type cooling tower. |  |
| b) | Explain the construction and working of pressurised water reactor. |  |
| c) | Draw the general layout of nuclear Power Plant and explain. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Functions of gas loop |  |
| b) | Diesel engine Power Plant cooling system |  |
| c) | Effect of variable load on Power Plant |  |
| d) | De-aeration process |  |
| e) | Site selection for hydal Power Plants |  |
| f) | Carry over and fog formation in cooling tower |  |

***********