## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC

Subject: Communication Systems-II(4144) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define angle of tilt. Explain briefly why there is a limit on the distance of communication using ground wave propagation. |  |
| b) | Give a brief comparison between coaxial cable and flat twin pair. |  |
| c) | Define the following: i) Polarization ii) Directive gain iii) radiation resistance. |  |
| d) | Briefly explain Shannon Hartley theorem. Give its use. |  |
| e) | Explain the term ‘granular noise’ in brief. What is its significance? |  |
| f) | Differentiate between FSK and PSK. |  |
| g) | Give the advantages of TDM over FDM. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is sky wave propagation? With the help of suitable diagrams explain why it is easier to achieve during the night compared to day. |  |
| b) | A 150m antenna, transmitting in the MF range at 2 MHZ has an antenna current of 5A. Find the voltage received by a receiving antenna 50Km away, with a height of 2.5m. |  |
| c) | What is a transmission line stub? Explain the procedure for matching of the transmission line by using a stub. Draw relevant figures. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various feed mechanisms used for a parabolic reflector antenna? Draw a diagram and explain any one of them. |  |
| b) | With the help of a neat sketch, explain the working of a TV antenna used for reception. Also draw its radiation pattern. |  |
| c) | Draw a diagram and briefly explain the TE10 mode of propagation in a rectangular waveguide. Derive the relation between group velocity and phase velocity. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a block diagram, explain any one method of generation of PWM. Also draw the waveforms. |  |
| b) | Define quantization error in PCM. With the help of relevant diagrams explain how it is overcome. |  |
| c) | With the help of a neat block diagram explain the working of a Delta modulation transmitter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat block diagram explain the working of a QPSK receiver. |  |
| b) | Draw a neat block diagram and explain the working of a FSK transmitter. |  |
| c) | Explain how FDM can be used to achieve multichannel communication. Draw the block diagram of such a system for five channels. | …..2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | VSWR |  |
| b) | Cut off wavelength in waveguide |  |
| c) | Pulse position modulation |  |
| d) | Application of TDM |  |

***********