## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg.

Subject: Machine Drawing-II(4035) [Old]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A shaft and a hole are given as below: +0.25 +0.10 +0.12 +0.00 Shaft : 25 Hole : 25 Determine: i) Maximum allowance ii) Minimum allowance iii) Type of fit. |  |
| b) | Draw the sectional representation and the appropriate symbols for the following forms of welds: i) Convex double V-butt weld ii) Plug welding or slot welding. |  |
| c) | Show weld representation symbol with significance of each entity involved. |  |
| d) | Draw the double line representation for the following pipe fittings: i) Cap ii) Tee. |  |
| e) | Draw the symbols for the following features, which are controlled in geometrical tolerancing: - Circularity ii) Profile of surface iii) Angularity |  |
| f) | Draw a table in which Roughness grade, roughness numbers and the corresponding roughness symbol is shown for the following values of roughness grades (Ra in µm) Ra = 50, Ra = 12.5, Ra = 0.4, Ra = 0.05 |  |
| Q.No.2. | Draw neat proportionate freehand sketches of any two of the following: - Muff Coupling - V-belt pulley (Half sectional front view, top half in section) - Knuckle joint | 2 x 6 = 12 |
| Q.No.3. | Answer any one of the following Questions: | (12) |
| a) | Fig. No.1 shows the details of a pin type flexible coupling. Draw the following views of the assembly according to the third angle method of projection: - Half sectional front view - Right hand side view |  |
| b) | Fig. No.2 shows the assembly of foot-step bearing. Draw the following parts in disassembly: - Body of the bearing - Bush - Disc - Shaft |  |
| Q.No.4. | Fig. No.3 shows the two orthographic views of the base plate of a ‘Drill Jig’. Redraw the views and incorporate following details: - Tolerance of g6 on thickness of 25mm - Bottom face of the base plate to be indicated as datum A. - Top face of the base plate is parallel to bottom face and to be within a tolerance of 0.02mm. -2- - Roughness on the inner surface of hole of diameters 54mm to be indicated by VV. - Axis of tapped hole of M12 to be co-axial with the axis of hole of 17mm within  0.08mm. | (12) …..2/- |
| Q.No.5. | A connector is shown in fig. No.4. Answer the following questions pertaining to this figure. - What is the size of the knurled portion of the component? - What is the size of the Key way? - What is the depth of the drilled hole? - Explain the note 4 x Ø6. - What is the meaning of 4 x 45. - Explain the meaning of the surface symbol at 50f7. - Explain 50f7. - What type of tolerance is given on  16mm? - What is the length of internal thread? - Explain the note ‘X(5:1)’. | (10) |
| Q.No.6. | Answer the following Questions: | 2 x 7 = 14 |
| a) | Fig. No.5 shows a double line representation of a pipeline alongwith various fittings assembled together. Draw the single line schematic representation of the same. |  |
| b) | Fig. No.6. shows the view of a machine part to be fabricated by welding process. Draw any two views. Select suitable types of welds and indicate the same through the appropriate symbols. |  |