## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Elementary Ship Design(SH701)/(5331) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Trading Pattern |  |
| b) | Parent vessel in ship design |  |
| c) | Design features of bulk carrier |  |
| d) | Types of freeboard |  |
| e) | Breadth to draft ratio of a ship |  |
| Q.No.2. | Answer any one of the following Questions: | (12) |
| a) | What is meant by Gross tonnage and Net tonnage? How is it computed? Explain the influence of Gross tonnage and Net tonnage on a vessel. |  |
| b) | What is a general arrangement plan? Draw the typical general arrangement plan of a bulk carrier. Explain requirement of crew accommodation. |  |
| Q.No.3. | A ship owner requires a vessel meeting the following requirements: - Dead weight = 3250 tonnes - Service speed =12.5 knots Determine the length, breadth and draft of the vessel required to achieve the above, given: - Cd=0.912−0.19VL 2. L=6.7VV+22Δ13 3. CB=1.137−0.33VL 4. B=L9+3 V is in Knots and L in metres | (12) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by main particulars of a ship? Give an example of main particulars of a vessel. |  |
| b) | What ar the various components of the Lightship weight? How are they determined? |  |
| c) | Explain how the machinery weight of a vessel can be estimated in preliminary design stage. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain a typical technical specification of a merchant vessel prepared for the construction of a vessel. |  |
| b) | Estimate the lightship weight of a vessel of service speed 15.5 Knots of dimensions 155m x 23.3m x 13.3m given the following: - Hull weight of a basis vessel of dimensions 147m x 24m x 12.8m is 6500 tonnes. - Wood and outfit weight of the basis vessel is 350 tonnes. - Machinery weight is 1100 tonnes. |  |
| c) | For a bulk carrier of length 220m, breadth 31.5m and draft 14.2m estimate the following i) KB ii) KG iii) BM. | ….2/- |
|  | -2- |  |
|  | only for students of revised scheme: |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the purpose of stability booklet? |  |
| b) | Explain briefly the criteria for choosing low speed engine. |  |
| c) | List the life saving appliances onboard a passenger ship. |  |
|  | only for students of old scheme: |  |
| Q.No.6. | Explain how the lines of a vessel are obtained given the sectional area curve of a vessel along with the associated load waterline. | (12) |

***********