## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical/Production Engg.

Subject: Tool Engineering-I(Cutting Tools)[5033]

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List down the factors that deteriorate surface finish. |  |
| b) | Why are discontinuous chips preferred over continuous chips? |  |
| c) | On what factors does the choice of feed and speed depend in machine tool operations? |  |
| d) | What are the causes of failure of cutting tools? |  |
| e) | What do you mean by side relief angle? |  |
| f) | What are the contents of 18:4:1 HSS Tool material? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw Merchants circle diagram for cutting forces on orthogonal cutting. |  |
| b) | During a metal cutting operation it was observed that shear plane angle was 35. The length of travel (i.e. uncut chip) was 150mm. what is the length of chip you will except? |  |
| c) | Define tool life. Discuss different factors affecting tool life. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the effects of tool geometry on metal cutting characteristics? |  |
| b) | If the relationship for HSS tools is VT1/8 = C1, and for tungsten carbide tools VT1/5 = C2 and assuming that at a speed of 25 m/min, the tool life was 3 hours in each case, compare their cutting lives at a speed of 32 m/min. |  |
| c) | Explain the constructional features of flat and circular form tools. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Edge wear. |  |
| b) | With neat sketches explain two types of milling operations. |  |
| c) | What are the constituents of cutting tool material? State the advantages and disadvantages of each material. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Name different cutting tool materials and explain the properties and applications of each one. |  |
| b) | State the qualities of a good cutting fluid. |  |
| c) | How is soluble emulsion prepared? |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Storage and disposal of cutting fluid |  |
| b) | Abrasive wear |  |
| c) | Taps |  |
| d) | UCON |  |

***********