## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(4104)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Find the voltage Vxy for the circuit shown in fig.1(a). |  |
| b) | Derive an expression for the power consumed in a R-L circuit. Draw the waveforms of voltage, current and power consumed. |  |
| c) | Draw the bridge for measurement of frequency. Give its equation at balance & at least two advantages. |  |
| d) | The arms of an a.c. Maxwell’s bridge are arranged as follows: AB and BC non-inductive resistors of 100 each. DA is a standard variable in reactor L1 of resistance 32.7 and CD comprises of a standard variable resistor R in series with a coil of unknown impedance. Balance was obtained with L1= 47.8 mH and R=1.36. Find resistance and inductance of the coil. |  |
| e) | Derive the expression of transient voltage for a R.C. (D.C) circuit. |  |
| f) | Define time constant for a R.L. (D.C) circuit. Give its significance. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For the circuit shown in fig.2(a), find the equivalent resistance across terminals A and B. |  |
| b) | By superposition theorem, find the current through 4 resistor for the circuit shown in fig.2(b). |  |
| c) | Find the Thevenin’s equivalent circuit across terminals A and B for the circuit shown in fig.2(C). |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two coils A and B are connected in series across 240V, 50Hz supply. The resistance of coil A is 5 and inductance of coil B in 0.15H. IF the input power from the supply is 3 KW and 2 KVAR, find the inductance of coil A and resistance of coil B. |  |
| b) | A voltage of  = 100 sin 314t is applied to circuit consisting of a 25 resistance and 60 F capacitor in series. Determine the - expression for the current at any instant. - Power consumed by the circuit. - The potential difference across the capacitor at the instant when the current is half of its maximum value. |  |
| c) | 3 bulbs A – 230V, 40W; B – 230V, 60W and C – 230V, 100W are connected in parallel across 230V, 50Hz, A.C. 1 –phase supply. Calculate i) Resistance of each bulb ii) The supply current. If all the bulbs are connected in series across the same supply, calculate the current drawn by them. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Prove that the resonance frequency in a parallel resonance circuit is f=12π1LC−R2L2Also derive expression for the quality factor. |  |
| b) | Define the following terms and gives their significance i) Quality factor, ii) Bandwidth & iii) Resonance curve. |  |
| c) | A coil having an inductance of 50 mH and resistance of 10 is connected is series with 25F capacitor across 200V a.c. supply. Calculate current at resonance, value of Quality factor, bandwidth and resonance frequency. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Prove that the line voltage is equal to 3 times the phase voltage in a star connected system. |  |
| b) | Three non-inductive resistances of 100  are connected in i) star & ii) Delta across a 3 – phase 400 V supply mains. Find the line current, power factor and power in each case. If one of the resistance is disconnected from the supply, calculate the line current, power factor & power in each case. |  |
| c) | Three identical inpedances are connected in delta across a 3-phase 400 V supply. The line current is 35A and the total power taken from the supply is 14.4 KW. Calculate the resistance and reactance of each impedance. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the Hay’s bridge and give the expressions for unknown quantities at balance. What are its advantages and disadvantages? |  |
| b) | The unbalanced circuit of fig.6(b) in connected across a 3-phase supply of 400V. Calculate the currents and phase voltages. Phase sequence in RYB. |  |
| c) | Define zero, positive and negative sequence components. Determine the values of the symmetrical components of a system of currents IR= 0+j120 A, Iy = 50-j100A and IB = 100 – j50A. |  |