## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: A.C. Machines(4113)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Derive the expression for frequency of rotor emf of 3 induction motor under running condition. |  |
| b) | Draw the power stage diagram of induction motor. |  |
| c) | Discuss the effect of increase in output power of Induction motor on i) speed ii) power factor iii) efficiency iv) stator current. |  |
| d) | Why 1 induction motors are not self starting? List the different methods of making 1 induction motor self starting. |  |
| e) | Calculate distribution factor for 48 slots, 4 pole single layer 3 winding. |  |
| f) | Discuss the different losses taking place in alternator. |  |
| g) | Define the term running torque, pull in torque. |  |
| h) | Discuss the advantages of parallel operation of alternator and give the conditions required to be fulfilled for parallel operation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 3 induction motor runs at almost 1000 rpm at no load and 950 rpm at full load when supplied with power from a 50 Hz, 3 line. Calculate i) number of motor poles ii) % slip at full load iii) frequency of rotor voltage iv) speed of rotor field with respect to rotor v) speed of rotor with respect to stator field vi) rotor frequency at slip of 10%. |  |
| b) | Draw and explain Torque-slip and Torque-speed characteristics of 3 induction motor. What is the effect of rotor resistance variation on these characteristics? |  |
| c) | Derive relationship between: - full load torque and maximum torque - Starting torque and maximum torque. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 3, 6 pole, 50Hz induction motor develops 3700 watts at 950 rpm. What is the stator input if stator loss is 300 watt? |  |
| b) | Compare between squirrel cage and slip ring induction motor. |  |
| c) | Compare synchronous motor and induction motor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of Star-Delta connection in starter. With a neat control circuit diagram. Explain the working of automatic Star-Delta starter. |  |
| b) | Explain capacitor split phase method of starting 1 induction motor in detail. |  |
| c) | Discuss the construction of alternator. |  |
| d) | Draw and discuss the phasor of alternator on load for i)unity power factor load ii) lagging power factor load iii) leading power factor load. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A 3,  connected, 1000 KVA, 11000 volts alternator has rated current of 52.5 Amps. The ac resistance of winding per phase is 0.45 . The test results are as follows: - open circuit test:- field current = 12.5 Amps voltage between lines =422 volts. - Short circuit test:- field current =12.5 Amps line current = 52.5 Amps. Determine the full load voltage regulation of alternator i) at 0.8 power factor lag ii) 0.8 power factor lead. |  |
| b) | Discuss the effect of varying excitation on armature current and power factor in synchronous motor. |  |
| c) | Give merits, demerits, applications and main characteristics of synchronous motors. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the effect of following on the working of two alternators in parallel: - increase excitation of one of alternator - Change in speed of one of alternator - Unequal voltages. |  |
| b) | Explain construction and working of repulsion motor. |  |
| c) | Two 1 generators operating in parallel on a load have impedance Z , their emfs are E1 and E2 and synchronous impedances are Z1 and Z2. Determine terminal voltage and KW output of each machine given E1 = 100 volts, E2 = 110 volts, Z1= Z2 = 0.2 + j1 , Z = 3 + j4 . |  |

***********