## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT

Subject: Irrigation Engineering(4017)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Mention the various types of irrigation systems. Briefly explain each. |  |
| b) | Explain unit Hydrograph and its use. |  |
| c) | What are the points to be considered for locating the rain gauges? |  |
| d) | Explain Thiessen Polygon method of computing average rainfall, with suitable sketch. |  |
| e) | Sketch and label, section of a radial gate over the crest of a spillway. |  |
| f) | What are the various galleries in gravity dam? State the location and functions of each, with sketch. |  |
| g) | Define catchment, free and intercepted catchment. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive the relation between detla, duty and base period D=8.64BΔ. |  |
| b) | Write any six points on methods of improving duty of water. |  |
| c) | Water is released at the rate of 15 cumec at the head of a canal. If duty at the field is 1300 ha/cumec and loss of water in transit is 20%, find the area of the land that can be irrigated. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat cross section of a storage reservoir and label the following: Bed level, L.S.L., Max water level, H.F.L., Free board, Dead storage, Live Storage. |  |
| b) | Write any six points explaining how to minimize sedimentation or silt control of a reservoir. |  |
| c) | Write short notes on reservoir losses. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With sketches, explain elementary profile and practical profile of a gravity dam. |  |
| b) | Establish an expression for dimensions or forces for a gravity dam to be resistant to sliding and overturning. |  |
| c) | What are the energy dissipators and their functions? Show sketches of some energy dissipators. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat sections of typical homogeneous, cored and diaphragm earth dams. |  |
| b) | Explain the three classified types of failures of earth dams and detailed type of failures under each main type. |  |
| c) | Describe pointwise the preliminary surveys, geological and hydrological surveys required for a dam or reservoir project. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Advantages and disadvantages of bandhara irrigation |  |
| b) | Types and functions of Weirs for diverson works |  |
| c) | Methods of determining yield of a well by any one method |  |
| d) | Maintenance of canals |  |
| e) | Cross sections of canals in cutting, in embankment and in partly cutting and partly embankment |  |
| f) | Necessity of cross drainage works and types |  |

***********