## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Microprocessor Based Instrumentation(5114)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain what are SFR and list some of them. |  |
| b) | Explain the instruction cycle of 8051. |  |
| c) | List the rotate instructions and explain. |  |
| d) | Compare the following processors: 80386, 80486 and Pentium processor with reference to the data $ address lines, speed and compatibility. |  |
| e) | Give the comparison between microprocessor and microcontroller. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Name the four Segment Registers in 8086 and state their functions. |  |
| b) | Compare between the features of microcontroller 8751 and 8748. |  |
| c) | Explain the different addressing modes used in 8086. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Explain and write the format for TCON and TMOD special function registers. |  |
| b) | Draw and explain the internal RAM Memory organisation of 8051. |  |
| c) | For Port 2 and Port 3 explain I/O function with help of port pin configuration circuit diagram. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the internal and external interrupts provided by 8051. |  |
| b) | Explain the Special function registers associated with serial data transmission. |  |
| c) | Briefly explain the flags and program status word of 8051. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the following instructions with an example: - MOV X @ DPTR, A - MUL AB - SWAP A - ADDC A, @ Rp - ANL c,/b - DJNZ Rn, radd |  |
| b) | Explain the following program stating the contents of Registers A and B for every instruction. Also state the flags affected. - MOV A, # 7Bh - MOV 0F0h, #02h - MUL AB - MOV B,#0FEh - MUL AB |  |

………2/-

-2-

| c) | Explain in brief the conditional and unconditional jump instructions. |  |
| --- | --- | --- |
| Q.No.6. | Answer any two: | 2 x 6 = 12 |
| a) | Write a program to count the number of bytes which has data 08 H starting from RAM memory location 00H. The size of the block data byte is ten. |  |
| b) | Write a program to count the bits set in the bit addressable area of internal RAM memory location. Also count the bytes which have more than five ones in a byte. |  |
| c) | Explain the stack memory operations with relevant instructions. |  |

***********