## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Power Electronics & Drive Control(5102)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the circuit diagram of multiphase star rectifier and state its basic principle of operation. |  |
| b) | Comment on diodes with RC loads. |  |
| c) | Define (i) Peak inverse voltage (ii) latching current with respect to a thyristor. |  |
| d) | What do you understand by firing angle and conduction angle with respect to a thyristor? |  |
| e) | Explain in brief the operation of class A chopper. |  |
| f) | Draw the circuit diagram of a boost regulator and also sketch its waveforms. |  |
| g) | What do you understand by chopper controlled drives? Classify them. |  |
| h) | What do you mean by rheostatic braking control? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch the V.I characteristics of a thyristor and explain it in detail. |  |
| b) | Explain the operation of a thyristor firing circuit using PUT with circuit diagram and waveforms. |  |
| c) | Describe the series and parallel operation of thyristor with necessary sketches. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the operation of a single phase full wave bridge type controlled rectifier with neat circuit diagram and waveforms. |  |
| b) | With the help of circuit diagram and waveforms explain the phase control method used in ac voltage controllers. |  |
| c) | Write a short note on 1 cycloconvertor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit diagram of a three phase half wave ac voltage controller and explain its operation. Also sketch the relevant waveforms. |  |
| b) | Write a short note on regenerative braking control method used in chopper controlled drives. |  |
| c) | Explain the working of a single phase half wave converter drive with circuit diagram and neat waveforms. |  |
| Q.No.5. | Comment on any two of the following: | 2 x 6 = 12 |
| a) | Slip power recovery scheme |  |
| b) | Frequency control method |  |
| c) | Current control |  |
| Q.No.6. | Write a short note on any three of the following: | 3 x 4 = 12 |
| a) | Power Mosfet b)Parallel connected diodes |  |
| c) | Solid state relays d)Current protection in power devices |  |
| e) | Free wheeling diodes |  |

***********