## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg.

Subject: Machine Drawing-II(4035)

# Time Duration: 4 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Attempt any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Calculate the limit sizes for a hole designated as  25H7. Following data is given: - Diameter 25mm lies in the range from 18 to 30mm. - Unit tolerance i=0.001D+0.453D(microns) - IT7 = 16i. |  |
| b) | Indicate the roughness grade symbols for the following roughness grade numbers. i) N12 ii) N10 iii) N8 iv) N2 |  |
| c) | Interpret the meaning of geometrical tolerances of the parts shown in fig 1. |  |
| d) | Indicate with symbols each of the following: - Machining finish with shaping to a surface roughness value of 3.2microns - Machining allowance of 3mm - Sampling length of 12mm - Surface to be electroplated. |  |
| e) | Draw the double line representation for the following pipe fittings. i) Plug ii) Cap iii)90 Elbow iv) Flange |  |
| f) | Draw the sectional representation and the appropriate symbols for the following form of welds. - Convex double V-butt weld - Plug welding or slot welding - Single J-butt weld - Single U-butt weld |  |
| Q.No.2. | Draw neat proportionate free hand sketches of any two of the following: | 2 x 6 = 12 |
| a) | Knuckle joint (sectional F.V and Top view) |  |
| b) | Solid flanged coupling |  |
| c) | V-belt pulley | ……2/- |

-2-

| Q.No.3. | Attempt any one of the following: | (12) |
| --- | --- | --- |
| a) | Fig No.2 shows the details of a pin type flexible coupling. Draw the following views of the assembly according to the Third angle method of projection: i)Half sectional front view ii)Right hand side view. |  |
| b) | Fig No.3 shows the assembly of Foot-step bearing. Draw the following parts in disassembly: i)Body of the bearing ii) Bush iii) Disc iv) Shaft. |  |
| Q.No.4. | Fig No.4 shows the two orthographic views of the sheave of an eccentric. Redraw the views and incorporate the following details. i) Cylindrical surface of  200mm to be machined and roughness to be indicated by  ii) Tolerances of H7 on  80mm iii) Roughness on the inner surface of hole of  80mm to be indicated by  iv) Axis of hole of  80mm to be indicated as datum A v)Face on the left hand side to be perpendicular to datum within 0.1mm vi)Tolerance of K7 on the width of the keyway | (12) |
| Q.No.5. | Attempt the following: |  |
| a) | Redraw the view given in fig No.5 (by neat proportionate free hand). Plates marked as 1,2,3 and 4 are to be welded as mentioned below. Use proper welding symbols to indicate the following welds for the joints. - Piece 1 is welded to piece 2 by single J Butt - Piece 2 is welded to piece 3 by all round fillet weld arc welding - Piece 1 is welded to piece 4 by square butt weld-finished flush on top | (6) |
| b) | Fig. No 6 shows a piping layout in the form of a block diagram with the names of various fittings mentioned in the blocks. Draw the single line diagram for the same. | (6) |
| Q.No.6. | Study the working of a Gear shown in fig.No.7 and answer the following questions. - What is the size of the bore of the gear? - What is the roughness of the inner surface of the bore of the gear? - What is the width of the keyway? - What is the surface roughness of the side faces of the keyway? - What is the meaning of 1.6 x 45? - What is the manning of geometrical tolerance indicated at ‘a’? | (12) ….3/- |