## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mining Engg.

Subject: Mine Ventilation(4297)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the standards of Mine ventilation. |  |
| b) | Differentiate between homotropal and antitropal ventilation. |  |
| c) | What are the advantages of exhaust system over forcing system of ventilation? |  |
| d) | How does an evasee reduce the ventilation cost? |  |
| e) | What are the advantages of splitting? |  |
| f) | Give the objectives of ventilation survey. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the physical, chemical and physiological properties of methane. |  |
| b) | i)What is measured with kata thermometer? What different parameters does it include? ii)How does humidity affect the performance of an underground coal mine worker? |  |
| c) | Explain the principle and working of a methanometer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the purpose & construction of air crossing in an underground mine. |  |
| b) | Describe the principle & working of an axial how fan with a near sketch. |  |
| c) | Air temperature in a D.C. shaft 465m deep is 30C and that in the U.C. shaft of same depth is 37C. what assistance expressed in H.P. does this difference in temperature render when quantity of air passing through the shaft is 100 m3/s? (Assume average barometric pressure in D.C. shaft to be 750 mm of Hg) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure of conduction pressure survey by inclined manometer. |  |
| b) | i)Write short notes on rigid ducts. ii)With a neat sketch explain overlap method of ventilating a heading by auxiliary fan. |  |
| c) | Calculate the water guage developed and the quantity delivered by a backward bladed centrifugal fan having the following specifications: Fan dia = 3.8m RPM = 350 Width at periphery = 1.7m Blade angle = 45 Velocity of how = 4.5 m/s Air density = 1.2Kg/m3 |  |

……2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | The velocity of air in a 6m dia. D.C shaft 300m deep is 4 m/s. Calculate i) the quantity of air entering the mine per minute ii) the H.P lost to overcome the frictional resistance of the shaft. Assume coefficient of friction to be 0.00161 mm w.g for a velocity of 1m/s. |  |
| b) | i)Derive an equation for the equivalent resistance when roadways are connected in parallel? ii)What is the equivalent resistance of two roadways of equal resistance connected in parallel? | (4) (2) |
| c) | Total quantity of air passing through a district of a mine is 25 m3/s with a w.g. of 25cm. It is now proposed to reduce the quantity of air flowing through the district to 20 m3/s. Calculate the size of the regulator that should be fitted in the district? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are the factors causing NVP? |  |
| b) | Write short notes on methane layering. |  |
| c) | Draw the characteristic curves of any one type of fan. What are the uses of characteristic curves? |  |
| d) | Explain the terms i) Neutral line of booster fan b) critical pressure of booster fan. |  |

***********