## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Production/Automobile Engg.

Subject: Machine Tools & Production Process-II(4060)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How many different types of vices are used in shaping machines? |  |
| b) | What are the various operations performed on a planer? |  |
| c) | Give classification of milling cutters. |  |
| d) | What is positive, zero and negative rake in a milling cutter? |  |
| e) | Why is a sensitive drilling machine called so? |  |
| f) | Draw a line diagram of a horizontal boring machine and label its different parts. |  |
| g) | Give specifications of a Slotter. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various shaping tools on a shaper? |  |
| b) | How can dovetail cutting be done on a shaper? |  |
| c) | In drilling operations, give applications of the following tools & accessories: i)Drill bit ii)Drill sleeve iii)Drill drift iv)Centre Drill. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Several 10 mm holes are to be drilled through a 15 mm thick piece of stainless steel. Cutting speed is 10 m/min and feed rate is 0.125 mm/revolution. How long will it take to drill each hole? |  |
| b) | Discuss briefly two of the tool holding devices used in boring machine operation. |  |
| c) | Draw a sketch of Jig boring machine and explain its main parts. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give two major differences between shaper and slotter. What is the use of index plate in a slotter? |  |
| b) | Explain slotter disc mechanism used in slotting machines? |  |
| c) | Draw sketch of open and cross bet drive of a planer mechanism. Explain its working. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the importance and functions of the following parts of a standard double housing planner. Draw sketches as required: i)Housing ii)Crossrail. |  |
| b) | Explain: i)Cam milling ii)Gear cutting operation on a milling machine. |  |
| c) | Suggests safety measures to be taken while working on machines such as a planner to avoid accidents. |  |
| Q.No.6. | Write short notes on any three of the following: | 3 x 4 = 12 |
| a) | “Feed” defined for the purpose of milling |  |
| b) | Right hand and left hand cutter for milling |  |
| c) | Universal vise for milling purpose |  |
| d) | Comparison between plain & universal milling |  |
| e) | Arbor assembly |  |

***********