## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Elementary Ship Design(5331)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Attempt any 5 questions

from Q.No.2 to Q. NO. 7.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Basis Vessel in ship design |  |
| b) | Dead weight |  |
| c) | Owner’s requirement |  |
| d) | Significance of sectional area curve |  |
| e) | Tonnage |  |
| f) | Cargo capacity |  |
| g] | Admirality co-efficient |  |
| Q.No.2.a) | Explain the stages of design with respect to design spiral. At what stage the general arrangement and lines plan prepared and why? | (6) |
| b) | How do canals, bridges, waterways and ports affect the feasibility study for ship design? | (6) |
| Q.No.3. | Estimate the dimensions of a dry cargo vessel of 12,000 tonnes DWT at a maximum draught of 7.5 m with service speed of 14 knots. | (12) |
|  | Given Cd = 0.67 |  |
|  | B = (L/9) + 6 |  |
|  | CB = 1.00 – 0.23 vL |  |
|  | Where v  knots |  |
|  | Cd  Deadweight co-efficient |  |
|  | L  Length in mtrs |  |
|  | B  Breadth in mtrs |  |
|  | CB  Block co-efficient. |  |
| Q.No.4.a) | Explain how the lightship weight of a vessel can be estimated in the preliminary design stage. | (6) |
| b) | A ship in loaded condition has a displacement of 16420 tonnes, draft 7.4m, CB =0.66, Cw = 0.767. If the length of the ship is L=120m, estimate the TPC and MCT at this loaded condition. Also estimate the draft for a displacement of 8800 tonnes. | (6) |
| Q.No.5.a) | What is meant by a sectional area curve? Explain how sectional area curve can be modified to obtain a new position of LCB. | (9) |
| b) | Explain the stability requirement of newly designed ship. | (3) |
| Q.No.6. | Draw a typical GA plan of a merchant vessel. Indicate the important machinery, spaces and fitting on board the vessel. | (12) |
| Q.No.7.a) | What is free board? Is it a statutory requirement? Explain broadly how the minimum free board is computed? | (6) |
| b) | Explain a typical technical specification and contract signed between the ship owner and shipyard for construction of a vessel. | (6) |

***********