## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mining Engg.

Subject: Mine Surveying(5396)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State difficulties in Mine surveying. |  |
| b) | List out instruments used in correlation survey. |  |
| c) | Explain steps in stope surveying. |  |
| d) | Define any three: - Throw of fault 2. Hade of fault. 3. Dip of a ore body 4. Strike of an ore body. |  |
| e) | Draw convention of following as per CMR 1957 (second schedule): 1. Shaft 2. Inclined 3. Air crossing. |  |
| f) | State formula of calculating volume of coal in inclined seam. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain method of calculating mineral in stock pile. |  |
| b) | Explain method of calculating area from plotted plan. |  |
|  | ### OR |  |
| b) | A Coal heap standing on level ground has rectangular base 50 meters, 30 meters broad. The height of heap is 5 meters and angle of repose of coal is 45º. Calculate cubic content of the heap and weight of coal in the heap. Assuming 1m3 of coal weight is 1.3 tonnes. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | State use of EDM/Total station in open cast mines. | (4) |
| b) b) | Two seam of coal are 40m vertically apart with a dip of 20º to the horizontal. Calculate the length of a cross measure drift driven to connect them if i) Drift is level ii) If it is rising 1 in 8 towards the deep of seam iii)If it dipping 1 in 2 in the same direction of the seam. ### OR A roadways driven on the full dip of a seam which is 1 in 6 encounters a normal fault bearing at right angle to the roadways. The hade of fault is 20º from the vertical. A cross measure drift rising 1 in 4 driven on the line of the roadways produced recovery the seam 120m from the edge of bottom part along floor of drift. The direction and rate are the same on both sides of wall. Calculate throw of fault and width of barren ground. | (8) (8) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Explain role of mine surveying in opencast mine. | (5) |
| b) | Give classification of errors with example. | (5) |
| c) | List out plans used in surface mining. | (2) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Explain coplanning/traversing through main entries of correlation. | (5) |
| b) | Explain various steps in correlation. | (3) |
| c) | Explain method of stope surveying in cut & fill method of stope. | (4) ..2/- |

-2-

| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Purpose of stope surveying |  |
| b) | Traversing in stope surveying |  |
| c) | Methods of plotting mine plans |  |
| d) | Storage of mine plans and surveying instruments |  |

***********