## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Fabrication Technology & erection Engg.

Subject: Materials & Processes(4238)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the commercial processes for making steel. State the advantages of electric furnace process of making steel. |  |
| b) | Classify plain carbon steel on the basis of carbon content and give their applications. |  |
| c) | What is high speed steel? Mention the important types of H.S.S. |  |
| d) | Differentiate between Gray and White cast iron. |  |
| e) | What is the significance of tensile test? How would you measure percentage elongation? |  |
| f) | What is the need for relieving the stresses of weldments? |  |
| g) | What are the factors promoting hot cracking of weld? |  |
| h) | What is the purpose of Annealing and how is it done? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a part of iron-carbon equilibrium diagram (up to 2 C) and explain its important microstructural constituents. |  |
| b) | Discuss the effect of low and high temperature on the mechanical properties of steel. |  |
| c) | Mention the important alloying elements in steel. State their effect on the properties of steel. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the different types of tests used for identifying the metals and alloys. |  |
| b) | Explain the metallurgical changes which occur due to absorption of gases by welds. |  |
| c) | Explain the concept of Epitaxial growth in weld metal solidification. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why steel is tempered? What are the effects of tempering on the mechanical properties of steel? |  |
| b) | What do you mean by hardenability of steel? State the effects of carbon and alloying elements on hardenability. |  |
| c) | Describe the process of Age hardening using Al-Zn system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the mechanism of work hardening. State its practical applications. |  |
| b) | Explain the principle of Forging and Extrusion with a neat sketch. |  |
| c) | Discuss the properties and applications of important refractory materials. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Heat resisting steel b) Alloy cast iron |  |
| c) | Vibratory stress-relief d) Weld decay |  |
| e) | Quenching media f) Thermocole |  |

***********