## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Mollier chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe the advantages of Solar energy. |  |
| b) | Define & explain higher & lower calorific value of fuel. |  |
| c) | State and explain Gay-Lussac’s Law. |  |
| d) | Define and explain Entropy of steam. |  |
| e) | What is the function of Air preheater? How does it affect efficiency of the boiler? |  |
| f) | How does the clearance volume affect the performance of the Air compressor? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A producer gas used as a fuel, has the following volumetric composition: H2=28%; CO=12%; CH4=2%; CO2=16% and N2=42%. Find the volume of air required for complete combustion of 1m3 of this gas. Air contains 21% by volume of oxygen. |  |
| b) | 0.1m3 of air at a pressure of 1.5 bar is expanded isothermally to 0.5m3. Calculate the final pressure of the gas and heat supplied during the process. |  |
| c) | Explain the adiabatic process. How is this process different from polytropic process? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the internal energy of 1 Kg of superheated steam at a pressure of 10 bar and 280C. If this steam be expanded to a pressure of 1.6 bar and 0.8 dry, determine the change in internal energy. Assume specific heat of superheated steam as 2.1 KJ/KgºK. |  |
| b) | A steam power plant is supplied with dry saturated steam at pressure of 12 bar and exhausts into a condenser at 0.1 bar. Calculate the Rankine efficiency. |  |
| c) | Explain with a neat sketch the working of a La-Mont boiler. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the working of a Barrel Calorimeter. |  |
| b) | Find the enthalpy drop and final condition of steam when it is expanded isentropically from an initial pressure of 30 bar and 350ºC to a pressure of 1 bar. Use Mollier chart. |  |
| c) | Describe various types of draught system used in steam boilers indicating their main advantages. |  |

………2/-

-2-

| Q.No.5. | Write short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Modified Rankine cyle |  |
| b) | Axial Flow compressor |  |
| c) | Fusible plug |  |
| d) | After-cooling in air-compressor |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Compressed air motor |  |
| b) | Babcock & Wilcox boiler |  |
| c) | Bomb Calorimeter |  |
| d) | Universal gas constant |  |
| e) | Geothermal energy |  |

***********