## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Find the force exerted by a jet of water of diameter 7.5 cm on a stationary flat plate, when the jet strikes the plate normally with a velocity of 20 m/s. |  |
| b) | Define - Hydraulic efficiency - Mechanical efficiency - Volumetric efficiency of a turbine. |  |
| c) | What do you mean by cavitations of a centrifugal pump? |  |
| d) | Draw ideal indicator diagram. |  |
| e) | Draw a neat sketch of an internal gear pump. |  |
| f) | Write down the symbols for the following pneumatic elements: i) Shuttle valve ii) Safety valve iii) Double acting cylinder. |  |
| g) | Write down the symbols for the following hydraulic elements: - Check valve - Direction control valve - Flow control valve. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Pelton wheel is working under a head of 300m and develops 736 KW at 200 rpm. Find - Jet diameter - Bucket sizes & - Number of buckets required. Given: Hydraulic efficiency as 85% and Cv as 0.98. |  |
| b) | Draw a neat sketch of Governing of Pelton turbine and label all components of it. |  |
| c) | Write a short note on cavitations of a turbine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A centrifugal pump having outer diameter equal to two times the inner diameter and running at 1000 rpm works against a total head of 40m. The velocity of flow through the impeller is constant and equal to 2.5 m/s. The vanes are set back at an angle of 40 at outlet. If the outer diameter of the impeller is 50 cm and width at outlet is 5 cm, determine: - Vane angle at inlet - Work done by impeller on water per second and - Manometric efficiency. |  |
| b) | A three stage centrifugal pump has impeller 40 cm in diameter and 2.5 cm wide at outlet. The vanes are curved back at the outlet at 30 and reduce the circumferential area by 15%. The manometric efficiency is 85% and overall efficiency is 75%. Determine the head generated by the pump when running at 12000 rpm and discharging 0.06 m3/s. Find the shaft power also. |  |
| c) | Derive the equation for minimum speed for starting a centrifugal pump. | ….2/- |

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A single acting reciprocating pump, running at 60 rpm delivers 0.53 m3 of water per minute. The diameter of the piston is 200 mm and stroke length 300 mm. The suction and delivery heads are 4m and 12m respectively. Determine - Theoretical discharge - Coefficient of discharge - Percentage slip iv) Power required to run the pump. |  |
| b) | Explain with neat sketch the function of air vessels in reciprocating pump and the saving in work done by using vessels? |  |
| c) | Briefly explain the classification of reciprocating pump. Define slip and Negative slip. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the operation of quick exhaust valve in pneumatic circuits. |  |
| b) | Write a short note on maintenance of hydraulic control systems. |  |
| c) | A Kaplan turbine runner is to be designed to develop 7357.5 KW shaft power. The net available head is 5.50m. Assume that the speed ratio is 2.09 and flow ratio is 0.68 and overall efficiency is 60%. The diameter of boss is 1/3rd of the diameter of the runner. Find the diameter of the runner, its speed & specific speed. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a F.R.L. unit. |  |
| b) | Explain with neat sketch construction, working and applications of rotary vane pump. |  |
| c) | Draw constant head curves of hydraulic turbines. |  |

***********