## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(4339)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) Question No.1 is compulsory. Attempt any five from

the remaining questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Fraud’s law of comparison |  |
| b) | Ship Correlation Factor (SCF) |  |
| c) | ITTC friction Line (1957) |  |
| d) | Wake |  |
| e) | Stern tube |  |
| f) | Super cavitating propellor |  |
| Q.No.2. | Answer the following: | 2 x 6 = 12 |
| a) | Describe the procedure for conducting model test in an experiment towing tank. What data is obtained from the test and how is it used. |  |
| b) | A model of ship is towed in fresh water has total resistance of 67N. The length of model is 5.2m, wetted surface area is 4.25m2 and speed of towing is 1.8 m/sec. The ship is of length 131m and wetted surface area is 2740m2. Determine the total resistance of ship. Kinematic Viscosity = 1.14 x 10-6 m2/sec FW Kinematic Viscosity = 1.19 x 10-6 m2/sec SW. |  |
| Q.No.3. | Answer the following: | 2 x 6 = 12 |
| a) | Explain the geometry of a screw propeller with neat sketches showing all required views. |  |
| b) | A propellor has a pitch of 5.5m. When turning at 80 rev/min the speed is 13.2 knots, speed of advance 11 knots, propellor efficiency is 70% and delivered power 3000 KW. Calculate i) Real slip ii) Wake fraction iii) Propellor Thrust. |  |
| Q.No.4. | Answer the following: |  |
| a) | Describe the purpose and procedure of conducting open water test. What information is obtained from the test? | (6) |
| b) | Explain what is meant by delivered power and thrust. | (3) |
| c) | Describe in brief what is meant by water lubricated stern tube. | (3) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Discuss the need for boring stern tube in place after welding. | (5) |
| b) | Explain the types of resistances. Which type of resistance is dominant in i) Inland vessel ii) Medium speed sea going vessel iii) High speed sea going vessel. | (7) |

………2/-

-2-

| Q.No.6. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | The resistance test of a vessel of length 67.25m has been conducted in a towing tank. Length of model = 1.77m. Following are the results of the resistance test of the model. Speed (m/s); 0.51; 0.59; 0.66; 0.75; 0.82. Resistance(Kg); 0.072; 0.093; 0.122; 0.160; 0.220. Determine the total resistance of ship of length 67.25m at a speed 7.5 knots. fmodel = 1.728 SW fship = 1.418 SW. | (8) |
| b) | Define the following propellor terms - Propellor cavitation - Propellor Rake. | (4) |
| Q.NO.7 | Answer the following Questions: |  |
| a) | How required rpm of propellor is achieved with medium speed engine? | (4) |
| b) | Discuss the advantages of oil lubricated stern tube over water lubricated stern tube. | (4) |
| c) | Explain the meaning KORT NOZZLE propulsion. | (4) |

***********