## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Chemistry of Welding Processes(4231)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are amphoteric oxides? Give examples. |  |
| b) | State any three essential requirements of a good flux. |  |
| c) | Describe FCC crystal lattice. |  |
| d) | Write a note on “clad metals”. |  |
| e) | What are the three different types of soldering fluxes used? |  |
| f) | Briefly explain the three types of typical faults caused in welds due to gas absorption. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the process for production of oxygen by liquefaction of air. |  |
| b) | How is gasification of liquid oxygen carried out by using: i) Cold gasifiers ii) hot gasifiers. |  |
| c) | Describe the acetylene cylinder. Why there is need for porous filters in acetylene cylinders? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the structure & chemistry of oxy-acetylene flame. |  |
| b) | Explain how: - flame angle and - metal thickness are related to effective heat input in welding. |  |
| c) | What are the advantages of using flux covered electrodes? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why is it necessary to use flux in gas welding? What are the functions performed by the following flux ingredients: - slag forming ingredients - gas shielding ingredients - arc-stabilising ingredients - de-oxidising element. |  |
| b) | How are the electrodes manufactured by - dipping method - extrusion method. |  |
| c) | Briefly explain any four methods used to prevent gaseous contamination in welds. |  |

………2/-

-2-

| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Give a comparison between Helium and Argon w.r.t. i] ionization ii] density iii] safety of use & storage. |  |
| b) | How is gas quantity in a cylinder determined? |  |
| c) | How does the following impurities present in acetylene affect welding. i] acetone vapour ii] hydrogen-sulphide vapour. |  |
| d] | State the properties & uses of the following gases in welding. i] petrol & kerosene gas ii] propane & butane gas. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are the requirements of a flux used for welding Aluminium & its alloys? |  |
| b) | Explain how crystal size is related to mechanical properties. |  |
| c) | What are the effects of absorption of the following gases in weld pool: i] oxygen and ii] nitrogen. |  |
| d] | State any four advantages of using bare wire electrodes in welding. |  |

***********