## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Industrial Instrumentation(4218)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Identify the type of transducer: Pt 500 & sketch the characteristic. |  |
| b) | Explain the term Vena Contractor with reference to an orifice plate. State typical application of segmented orifice plate. |  |
| c) | Why is a 3 wire method used to connect an RTD to the bridge? Draw a diagram to show the 3 wire method. |  |
| d) | Compare the performance of orifice plate and venturi meter. |  |
| e) | Explain in brief the working of an E/P converter. State its application. |  |
| f) | Explain the dilution method for flow meter calibration. |  |
| g) | Why are quantity flow meters so called? State its classification and give example of each type. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a diagram explain the working of magnetic flow meter. Discuss the performance. |  |
| b) | Explain the principle of variable head flow meter. Draw the diagram of a differential pressure flow meter and state relationships for flow velocity and volume flow rate. |  |
| c) | Explain the working of a Doppler flow meter with the help of a diagram. State the advantages. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of working of a thermo couple. State law of intermediate temperature. Draw a thermocouple circuit with extension leads. |  |
| b) | Explain the working of an optical pyrometer with the help of a diagram. State its application. |  |
| c) | Explain the working of solid expansion type of thermometer. State advantages and disadvantages. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a Mcleod gauge, with the help of a diagram. State the pressure range. |  |
| b) | Discuss angularity and multiplication adjustment in C type Bourdon tube. Explain how backlash is minimized. |  |
| c) | Explain the working of inclined plane manometer and a barometer with the help of diagrams. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the working of Air purge system used for level measurement, with the help of a diagram. |  |
| b) | Discuss the servicing techniques of different types of level measuring instruments. |  |
| c) | Explain the working of capacitance level measurement system, with the help of a diagram. State advantages. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a help of a block diagram discuss the automatic control of a closed loop flow control system. |  |
| b) | Discuss the need and techniques of servicing different type of level measuring instruments. |  |
| c) | Explain the working of a DP cell with the help of a diagram. State its application. |  |

***********