## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg./P/A

Subject: Machine Drawing-I(4033)

# Time Duration: 4 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A straight line AB has its end point A 15mm in front of the VP and end point B is 50mm above the HP. The line is inclined at 45 to the H.P, while its front view is inclined at 60o to the XY line. Draw the projections of the straight line AB if its top view is 35mm long. Find the true length and the angle of inclination of the line with the V.P. |  |
| b) | Fig(1) shows the incomplete plan, elevation and part auxiliary plan. Complete the plan. |  |
| c) | An equilateral triangular lamina of side 25mm rests with one of its sides on H.P such that the surface of the lamina is inclined to H.P at 60 and the side on which it rests makes 30 to VP. Draw its projections by change of ground line method. |  |
| Q.No.2. | Answer the following Questions: | (10) |
| a) b) | A pentagonal pyramid with the edge of base 25mm and axis 50mm rests on its base with an edge of the base parallel to the V.P and near the V.P. It is cut by a section plane perpendicular to the V.P, inclined at 60 to the H.P and passing through a point on its axis, 15mm above the base. Draw the front view and the development of the lateral surface of the pyramid in one piece. ### OR Develop the lateral surface of a cylindrical three piece offset section shown in fig(2). The total length of the fitting is 800mm and the offset is 600mm. |  |

………2/-

-2-

| Q.No.3. | Answer the following Questions: | (12) |
| --- | --- | --- |
| a) b) | A square prism, side of base 45mm stands with its base on H.P and two of its rectangular faces are equally inclined to the V.P. It is completely penetrated by a horizontal square prism of side of base 35mm such that their axes are 8mm apart. The rectangular faces of the horizontal prism are equally inclined to H.P and its axis is parallel to V.P. Draw the lines of penetration. ### OR A vertical cone, base 60mm diameter and axis 70mm long is penetrated by a cylinder of 40mm diameter such that the axis of the cylinder is parallel to the axis of the cone. Draw the projections showing the curves of intersection when the axis of the vertical cylinder is 7mm from the axis of the cone and the plane containing the two axes is perpendicular to the H.P and inclined at 45 to the V.P. |  |
| Q.No.4. | Figure (3) shows two views of an object. Draw the sectional front view, the given top view and the right hand side view. | (16) |
| Q.No.5. | Answer the following Questions: |  |
| a) | With the help of a suitable freehand sketch, show the following types of sections: - full section - Removed section - Offset section - Revolved section. #### OR | (08) |
| a) b) | Give conventional representation for the following: - Glass - Bevel Gear - Ball bearing - Tension spring. Draw by double line method a 20mm diameter pipe showing the following parts: - Globe valve - Union - 90 Elbow bend. | (08) (09) |
| Q.No.6. | The object shown in fig (4) is to be fabricated by welding different pieces A, B, C, D and E in the proper relative positions. Draw the orthographic view of the object and indicate the proper welding symbols. | (08) |

……..3/-