## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Theory of Structures-II(4023)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | ### Answer any three of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Using Moment–Area method derive the expression for Slope and Deflection for a simply supported beam carrying UDL throughout. |  |
| b) | Define i) Short column ii) Long column iii) Buckling Load iv) Slenderness ratio. |  |
| c) | Prove that in a three hinged arch with UDL throughout Bending Moment at any point of arch is zero. |  |
| d) | Derive expression for Fixed End moments for a fixed beam carrying a point load at centre. |  |
| e) | Define Distribution factor and Carry over factor with examples of structures. |  |
| Q.No.2. | Answer any one of the following: | 1 x 12=12 |
| a) | A simply supported beam of I section has a span of 7 m with point loads of 30 KN, 60KN and 15 KN at a distances of 2m, 4m and 6m from left support. Determine Slope at both supports as well as deflection under each load. Take E = 200 KN/mm2 and Ixx=20458 cm4. |  |
| b) | A simply supported girder of span 10 m is carrying UDL of 60 KN/m for a length of 6m from left support. Also a point load of 50 KN at 2m from right support. Take EI= 30 x 10 12 N.mm2. Compute maximum deflection in beam. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A fixed beam of span 4 m carry UDL of 40 KN/m throughout along with a point load of 25KN at 1m from left support. Find FEMs and draw BMD and SFD. |  |
| b) | A fixed beam of span 8 m carry a UDL of 50 KN/m for left half span. Find Fixed End Moments and draw SFD and BMD. |  |
| c) | A fixed beam of span 5m carry point loads of 30 KN and 10KN at distances of 2m and 4 m from left end. Draw BMD and SFD of beam. |  |
| Q.No.4. | Answer any one of the following: | 1 x 12=12 |
| a) | Analyze a continuous beam ABC with AB =4m and BC =6m. A has fixed support while C is a simple support. AB carries an UDL of 30 KN/m throughout while BC has point load of 40KN at its centre. Assume EI uniform throughout. Draw SFD and BMD. |  |
| b) | A portal frame ABCD with height of column AB=CD=4.5m and beam BC=6m. Beam BC carries an UDL of 60 KN/m throughout. The flexural rigidity (EI) of columns is 1.5EI and beam is 2 EI. Find support moments and draw BMD, using moment distribution method. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A mild steel strut 50x50 mm in section has a length of 1000 mm with both ends fixed. Find Buckling load using Euler’s formula. Take E=200GPa. |  |
| b) | A hollow cast iron column of 140 mm outer diameter and 10mm thick. The length of column is 6 m with one end fixed and other hinged. Find safe load using Rankine’s formula. Take crushing stress 500 N/mm2 and Rankine’s constant  = 1/1600 and factor of safety as 5. |  |
| c) | An I section column ISHB 450 @ 907 N/m has height 7 m. The properties of column are Area =11189 mm2, Ixx =40349 cm4, Iyy=3045cm4. Find maximum and minimum load carrying capacity of given column. Assume appropriate end connection. Take E=2x105 N/mm2. |  |
| Q.No.6. | Answer any one of the following: | 1 x 12=12 |
| a) | A three hinged arch of span 40m with both supports at same level has a rise of 5m. It carries an UDL of 60 KN/m throughout the span along with two point loads 40KN/ at 5m from either supports. Find (i) Reaction at supports. (ii)Bending moment at a distance of 10m from left end. |  |
| b) | A three hinged arch of 10m span with central rise of 2.5m. It carries an UDL of 45 KN/m of length 5m placed symmetrically in the middle of arch. Find (i) Reaction at supports. (ii) Normal thrust and Radial shear at a distance of 4 m from left end. |  |

***********