## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT

Subject: Soil Mechanics(4012)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How proctor needle is useful in placement water content? |  |
| b) | Why study of ground engineering/soil mechanics is important? |  |
| c) | Distinguish between compaction and consolidation. |  |
| d) | What is the necessity of classification of soil? Give classification as per Indian standard. |  |
| e) | What do you mean by the term quick sand? |  |
| f) | What is the effect of water table on bearing capacity? |  |
| g) | What is bore hole log? |  |
| h) | Why folds occur in rock? Sketch and show various parts of folds. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Prove that γd=γ1+w with usual notations. |  |
| b) | Define the following: i) Porosity ii) Voids ratio iii) Air content iv) Density index. |  |
| c) | A soil has a specific gravity of 2.75 and porosity 38%. Calculate: i) Voids ratio ii) Dry density iii) Unit weight when 50% saturated iv) unit weight when fully saturated. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With aid of sketch describe the experimental procedure for determining permeability using constant head parameters test. |  |
| b) | In a falling head parameters test following observations were recorded: i) Length of soil sample = 8cms ii) c/s area = 52.5cm2 iii) permeability = 1.1 x 10-4 cm/sec iv) Height of fall = 36cm to 12cm Determine the diameter of the stand pipe. |  |
| c) | What do you mean by particle size? How will you ascertain it in the laboratory? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the assumptions made in the Rankines theory of earth pressure? |  |
| b) | In a laboratory compaction test the following observations were made: Water content (%) 5 12 18 24 30 Bulk Density 1.8 1.85 1.92 2.15 2.05 Calculate maximum dry density and optimum moisture content. |  |
| c) | What are the various field identification tests for fine grained soils? |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | How will you co-relate spring analogy to consolidation process? |  |
| b) | What do you mean by stabilization of soil? Briefly explain bitumen stabilization. |  |
| c) | What are the objectives of soil exploration? |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by unconformity? How does this differ from angular unconformity? |  |
| b) | What are joints? Draw sketches showing sheet jointing and column jointing. |  |
| c) | What are the advantages of triaxial test over direct shear test? |  |

***********