## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Transformers(4106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between core types and shell types transformer. |  |
| b) | Explain clearly the effect of leaking flux on the operation of a transformer. |  |
| c) | Explain the construction and function of a conservator in a transformer. |  |
| d) | Explain the advantages and disadvantages of an autotransformer over a two winding transformer. |  |
| e) | Briefly explain the various power losses occurring in a transformer. |  |
| f) | Explain briefly the construction of a welding transformer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different types of transformer core materials. |  |
| b) | Explain clearly the cooling method employed in a 11KV/440V distribution transformer. |  |
| c) | With a neat diagram explain the construction and operation of an OFF load tap changer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The number of turns on the primary and secondary of a single phase transformer are 350 and 38 respectively. If the primary winding is connected to a 2.2KV, 50Hz supply, determine: - The secondary voltage on No-load. - The primary current when the secondary current is 200A at 0.8 p.f. lagging. (No load current is 5A at 0.2 p.f. lagging) - The power factor of the primary current. |  |
| b) | A 30 KVA, 2400/120V, 50Hz transformer has a high voltage winding resistance of 0.1 and a leakage reactance of 0.22. The low voltage winding resistance is 0.035 and the leakage reactance is 0.012. Find the equivalent winding resistance, reactance and impedance referred to the - high voltage side and - the low voltage side. |  |
| c) | Draw and explain the phasor diagram of a transformer on No-load. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 100 KVA lighting transformer has a full load loss of 3 KW, the losses being equally divided between iron and copper. During the day, the transformer operates on full load for 3 hours, ½ load for 4 hours, and the output is negligible for the remainder of the day. Calculate the all day efficiency. |  |
| b) | Explain in detail voltage regulation of a transformer. |  |
| c) | Define efficiency for a transformer. Derive the condition for maximum efficiency. | …..2/- |

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the effect of failure of the cooling system on the performance of a transformer. |  |
| b) | Explain the condition that must be fulfilled for the successful parallel operation of 3-phase transformers. |  |
| c) | Describe the four phasor groups pertaining to 3 phase transformers. It is possible to use transformers of group I in parallel with transformers of group II? Justify your answer. |  |
| Q.No.6. | Write notes on any two of the following: | 2 x 6 = 12 |
| a) | High frequency transformers |  |
| b) | Condition for maximum and minimum voltage regulation |  |
| c) | Definition of per unit quantities |  |

***********