## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives(4115)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.I. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| 1) | Briefly explain the concept of electric drives. |  |
| 2) | Draw the block diagram of a typical closed loop drive. |  |
| 3) | Briefly explain the cooling process of a drive motor. |  |
| 4) | What are the important features to be satisfied by the drives to be used in cranes and machine tools? |  |
| 5) | Write short notes on drives used in sugar mills. |  |
| 6) | Write short note on drives used in textile mills. |  |
| Q.No.II. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | With a neat diagram, explain the four quadrantal operation of a motor driving a hoist load. |  |
| 2) | Explain the determination of equivalent reffered load torque and moment of inertia for drives having rotational motion. |  |
| 3) | A horizontal conveyor belt is moving at a velocity of 1.5m/sec and moves a load at the rate of 60,000 Kg/hour. The belt is 90m long and is driven by a motor with speed 960 rpm. Determine the equivalent rotational inertia at the motor shaft. |  |
| Q.No.III. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Explain the regenerative braking of a dc shunt motor. What are its limitations for a dc series motor? |  |
| 2) | With the help of neat diagram and characteristics, explain regenerative braking for a 3 phase induction motor. |  |
| 3) | With the help of neat diagram and speed torque characteristics, explain the speed control method of induction motor by pole changing method |  |
| Q.No.IV. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Explain the specifications to be specified for procurement of a drive motor. |  |
| 2) | Explain the effects of environmental factors on the performance & ratings of drive motor. |  |
| 3) | Using a heating cooling time curve, explain the heating and cooling process of drive motor. |  |
| Q.No.V. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | With the help of a neat diagram and example, explain the control circuit design for interlocking of two motors. |  |
| 2) | With the help of neat diagram and example, explain the control and power circuit design for remote control of an induction motor. |  |
| 3) | Write short note on limit switch and their applications. |  |
| Q.No.VI. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Discuss the choice of electric drive for lathe. |  |
| 2) | Compare regenerative and plugging methods of breaking. |  |
| 3) | Write short note on overload relay. |  |

***********