## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Computer Engg.

Subject: Computer Organisation-II(4222)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the advantages of CDROM? |  |
| b) | Write briefly on RAID level 1. |  |
| c) | Define hit-Ratio. |  |
| d) | State advantage and disadvantage of Associative memory. |  |
| e) | What is meant by memory segmentation? |  |
| f) | What is the role of IOP? |  |
| g) | What is meant by memory interleaving? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following disk performance parameters: (i) seek time (ii) rotational delay (iii) access time (iv) transfer time. |  |
| b) | Draw a diagram showing components of a multiple-platter hard disk and explain. |  |
| c) | With a diagram briefly explain hardware organisation of associative memory. Give an example to illustrate how information is searched in associative memory. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a diagram of one cell of associative memory and explain. |  |
| b) | Explain match logic giving equations. |  |
| c) | Explain set-associative memory with a diagram. What is its advantage? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of virtual memory. Give a diagram showing mapping of virtual address. |  |
| b) | A virtual memory has a page size of 1K words. There are eight pages and four blocks. The associative memory page table contains the following entries. Page; Block. 0; 3. 1; 1. 5; 2. 6; 0. Make a list of all virtual addresses (in decimal) that will cause a page fault if used by that CPU. |  |
| c) | Write a note on Base registers and Segment registers. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain CPU- IOP communication with a flowchart. |  |
| b) | Explain features of 8089 IOP. |  |
| c) | Explain I/O bus V/S memory bus. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a block diagram explain the organisation of SIMD array processor. |  |
| b) | Explain the operation of an instruction pipeline with a diagram. |  |
| c) | In certain scientific computations it is necessary to perform the arithmetic operation (Ai + Bi) (Ci + Di) with a stream of numbers. Specify a pipeline configuration to carry out this task. List the contents of all registers in the pipeline for i=1 through 6. |  |

***********