## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Production/Automobile Engg.

Subject: Machine Tools & Production Process-I(4050)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define a machine Tool. |  |
| b) | List the various types of Scrapers. |  |
| c) | What is the least count of a Micrometer screw? |  |
| d) | What is a Mallet? |  |
| e) | What is Pining of a file? |  |
| f) | Describe a Sine Bar. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the drill point geometry. |  |
| b) | Explain the use of a ‘V’ block while marking. |  |
| c) | With a neat sketch explain the construction of a Bench Vice. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a centre lathe and mark all important parts. |  |
| b) | What is a “Rest”? Explain. |  |
| c) | Explain the main differences between a three jaw self centering chuck and a four jaw independent chuck. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a cutting tool used for a facing operation. |  |
| b) | What is positive rake and negative rake angle of a cutting tool? Explain. |  |
| c) | Calculate the spindle speed if you are machining a work piece of 50 mm  & having a cutting speed of 20 m/min. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how taper turning operation is carried out on a lathe. |  |
| b) | What is a Mandrel? Explain with a sketch. |  |
| c) | Explain how a lathe is specified. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the various materials used to produce cutting tools. |  |
| b) | What are the desirable properties of an ideal cutting fluid? |  |
| c) | Explain the various types of cutting fluids used in a machine shop. |  |

***********