## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg.

Subject: Energy Management(5108)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by the term Energy Security? |  |
| b) | Briefly explain preliminary & detailed energy audit. |  |
| c) | Justify the statement ‘There is a need for Electrical load Management’. |  |
| d) | List down the ways by which watt loss in motors can be minimized. |  |
| e) | Write a short note on Energy Efficient transformer. |  |
| f) | Describe some of the important types of tariffs commonly used. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What does one understand by the term Energy conservation? Explain its importance. |  |
| b) | Briefly describe the energy strategy for the future. |  |
| c) | What is the importance of flattening load curve and explain methods to achieve it? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A factory operates at 0.8 pf lagging and has a monthly demand of 750 KVA. The monthly power rate is 8.50 per KVA. To improve the power factor 250KVA capacitors are installed in which there is negligible power loss. The installed cost of equipment is Rs. 20,000 and fixed charges are estimated at 10% per year. Calculate annual saving effected by the use of capacitors. |  |
| b) | Explain in brief the advantages and cost benefits of Pf improvement. |  |
| c) | A 3 phase, 5KW induction motor has a power factor of 0.75 lagging. A bank of capacitors is connected in Delta across the supply terminals and power factor raised to 0.9 lagging. Determine the KVAR rating of the capacitors connected in each phase. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain how motor loading can be determined. |  |
| b) | Explain the following terms: pump capacity, total developed head, pump efficiency and hydraulic power. |  |
| c) | Briefly explain the procedure for assessment of lighting systems and give the list areas for improvement. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What is the role of ballast? Compare conventional and Electronic ballast. |  |
| b) | What are maximum demand controllers and automatic power factor controllers? Justify their need in Electrical system. |  |
| c) | Explain - Localized switching - Daylight linked control - Time based control with reference to Energy efficient lighting controls. |  |
| Q.No.6. | Write short notes on any three of the following: | 3 x 4 = 12 |
| a) | Technical aspects of energy efficient motors |  |
| b) | Procedure for determination of Pump efficiency |  |
| c) | Demand curve & calculation of maximum demand |  |
| d) | Energy conservation act 2001 and its features |  |

***********