## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics Engg./EC

Subject: Television Engineering(5135)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is meant by compatibility in TV transmission? |  |
| b) | Explain the need for 625 line TV system to be capable of handling highest modulating frequency of 5 MHz for successful transmission and reception of picture details. |  |
| c) | In TV picture tube, explain - Function of aquadag coating - Spark gap protection. |  |
| d) | Indicate the possible fault locations and their causes in case the following: - Insufficient picture height - Ghost picture. |  |
| e) | Justify the choice of rectangular frame with width to height ratio = 4/3 for TV transmission and reception. |  |
| f) | In colour TV systems explain the terms: - Chrominance - Luminance - Hue. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch the details of horizontal blanking and sync pulse labelling the important regions. Why are front porch and back porch intervals provided before and after horizontal sync pulses? |  |
| b) | What is flicker? How interlace scanning reduces flicker? |  |
| c) | Explain VSB transmission. State its merits & demerits & comment on the correction required for its reception. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of suitable sketches how video signal is developed in a Vidicon camera tube. How is the vidicon different from an image orthicon and what are its special applications? |  |
| b) | Explain the construction of Vidicon tube. |  |
| c) | Sketch the sectional view of a picture tube and label all the parts. Explain how the electron beam is focused on the beam screen. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the merits and demerits of positive and negative amplitude modulation and justify the choice of negative modulation in most TV systems. |  |
| b) | Draw and explain the block diagram of monochrome TV receiver and label its various sections. |  |
| c) | What are the essential functions assigned to IF section of the receiver? Show by sketching output voltage v/s frequency response of the IF section, how VSB correction is carried out. | ……2/- |

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | How is the Y signal and colour difference signals developed from camera output? Why is (G-Y) difference signal not chosen for transmission? |  |
| b) | Explain the various conditions for compatibility between monochrome and colour TV. |  |
| c) | Write short notes on following: - Additive and subtractive mixing. - Parabolic reflector antenna. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch the block diagram of PAL-D coder. Mention some important features on PAL system and compare it with NTSC. |  |
| b) | Explain how by frequency interleaving the colour information is accommodated within the same channel bandwidth of 7 MHz. |  |
| c) | Write a note on pattern generator. |  |

***********