## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics Engg./EI/COMP/EC/MED/E&EE

Subject: Digital Electronics-II(4138)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the concept of BCD addition with the help of examples. |  |
| b) | Draw a clock waveform and define the following: - Rise-time - Frequency stability. |  |
| c) | Describe in brief, three differences between a SRAM and a DRAM. |  |
| d) | For a ladder type 5-bit DAC, calculate: - Output voltage for an input of 10102 - Full scale output voltage - Resolution. logic 0 = 0V and logic 1 = +5V. |  |
| e) | What are the buses or pins of a 4K x 8 EPROM? Write the size of each bus. |  |
| f) | Calculate the frequency and duty cycle of the output of an astable multivibrator using IC 555, which uses1K timing resistors and a 0.1 F timing capacitor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the logic diagram of a nibble multiplexer and explain its operation with the help of truth table. |  |
| b) | With the help of a logic diagram, describe a 1-to-8 demultiplexer circuit. Also write its truth table. How would you convert this into a decoder? |  |
| c) | Construct a 4-to-2 priority encoder. Write its truth table and explain how encoding is done by the encoder. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Construct a 4-bit binary ladder types DAC and explain the process of conversion of a digital signal into an analog signal. Write the formula for the analog output voltage and calculate the output voltage for an input 11012 with logic low as 0V and logic 1 as +10V. |  |
| b) | With the help of a neat diagram, describe the A/D converter which uses the successive approximation method. |  |
| c) | Draw the diagram and waveforms of a counter type ADC and describe its operation. List two points of difference between a counter type ADC and continuous ADC. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagram of a binary serial adder and explain how it performs addition. |  |
| b) | With the help of a diagram and waveforms, describe the application of a monostable multivibrator as a debounce circuit. |  |
| c) | For 2K x 8 EPROM, draw a diagram demonstrating how it is addressed, Explain how addressing a 2K x 8 RAM differ from addressing the EPROM. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Describe the structure of a PLA with the help of a neat diagram. Explain, giving an example, how data is stored in the PLA. |  |
| b) | Draw and describe the block diagram of an FPGA. |  |
| c) | Describe the use of a ROM as a PLD. Support your answer with a diagram depicting the ROM array and the supporting blocks. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw the logic diagram and truth table of: - Binary half adder - Binary half subtractor. |  |
| b) | Perform the following and write down the result: - 11012 x 10112 - 1101112  1012 |  |
| c) | Draw a TTL clock circuit and describe its operation. |  |
| d) | Given a 1K x 8 RAM, construct a RAM circuit which gives a capacity of 4K x 8 bits. |  |

***********