## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

#### Programme: Mechanical Engg./A

Subject: Theory of Machines(4039)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the law of belting. |  |
| b) | What is ‘Kinematic Pair’? Sketch and name any two types of kinematic pairs. |  |
| c) | What is the function of a Governor? Name any two types of Governors. |  |
| d) | Define the terms ‘Piston Effort’ & ‘Crank Effort’. |  |
| e) | On a freehand sketch of a tooth profile of a spur gear, identify the following: i) face ii) flank iii) Addendum iv) Clearance. |  |
| f) | Draw a sketch of a ‘Wedge Cam’. |  |
| g) | Define the terms ‘ Free vibrations’ & ‘Forced vibrations’. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | A part of a machine linkage is shown in Fig2(a). A and D are fixed centres. The crank AB rotates at 30 rad/sec clockwise. AB=10cm; BC=28cm; CD=24cm; DE=12cm; BG=16cm. For the position of the linkage shown, determine the velocity of point E. | (6) |
|  | ### OR |  |
| a) | In a 4-bar chair ABCD, AD is the fixed link 12 cm long. Crank AB is 3 cm long and rotates uniformly at 100 rpm clockwise while the link CD is 6 cm long and oscillates about D. Link BC is equal to length AD. Find the angular velocity of link DC when angel BAD is 60. | (6) |
| b) | In a slider crank mechanisam the line of stroke of the slider A is offset downwards a perpendicular distance of 6 cm from the crank centre C. The crank CB which is 24 cm long revolves clockwise at a speed of 250 rpm. The connectivity rod AB is 108 cm long. If a line DC, drawn from C and parallel to the line of stroke, makes 60 with CB, find the velocity of the slider A. | (6) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A screw jack spindle moving in a fixed nut has single start square threads on it. The pitch of the threads is 1 cm and outside diameter of the spindle is 5.5 cm. The load which does not rotate is carried on a swivel head whose bearings is 9 cm. If the coefficient of friction for thread and nut is 0.15 and that between swivel head and spindle is 0.12, find the load that can be lifted by applying a force of 20 Kg at the end of a 40 cm long lever. What is then the efficiency of lifting? |  |
| b) | The thrust on the propeller shaft of a marine engine is taken up by 8 collars whose external and internal diameters are 660 mm and 420mm respectively. The thrust pressure is 0.4 MN/m2 and may be assumed uniform. The coefficient of friction between the shaft and collars is 0.04. If the shaft rotates at 90 rpm, find (i) total thrust on the collars (ii) power absorbed by friction. |  |
|  |  | ……2/- |
|  | -2- |  |
| c) | A conical pivot bearing 150 mm in diameter has a cone angle of 120. If the shaft supports an axial load of 20 KN and the coefficient of friction is 0.03, find the power lost in friction when the shaft rotates at 200 rpm, assuming (i) uniform pressure and (ii) uniform wear. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | 2.5 KW of power is transmitted by an open-belt drive. The linear velocity of the belt is 2.5 m/sec. The angle of lap on the smaller pulley is 165. The coefficient of friction is 0.3. Determine the effect on power transmission if the initial tension in the belt is increased by 8%. | (6) |
|  | ### OR |  |
| a) | An open belt 100 mm wide connects two pulleys mounted on parallel shafts with their centres 2.4 m apart. The diameter of the larger pulley is 450 mm and that of the smaller pulley is 300 mm. The coefficient of friction between belt and pulley is 0.3 and the maximum stress in the belt is limited to 14 N/mm width. If the larger pulley rotates at 120 rpm, find the maximum power that can be transmitted. | (6) |
| b) | Explain with the help of a neat sketch the construction & working of a Hartnell Governor. | (6) |
| Q.No.5. | Answer the following Questions: |  |
| a) | The following data relates to a cam profile in which the follower moves with uniform acceleration and deceleration during ascent and SHM during descent. Minimum radius of cam= 25mm Roller radius = 7.5mm Lift = 28 mm Angle of ascent = 60 Angle of decent = 90 Angle of dwell between ascent & descent = 45 Speed of cam = 200 rpm. Draw the profile of the cam and determine the maximum velocity of the inline follower during the outstroke. | (9) |
| b) | State the Law of Gearing. | (3) |
|  | ### OR |  |
| b) | With the help of sketches highlight the features of a Compound Gear train and a Reverted Gear train. | (3) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The simple band brake shown in fig 6(a) is applied to a shaft carrying a flywheel of mass 400 kg. The radius of gyration of the flywheel is 450 mm and runs at 300 rpm. If the coefficient of friction is 0.2 and the brake drum diameter is 240 mm, find: i) the torque applied due to a hand load of 100 N ii) the number of turns of the wheel before it is brought to rest. |  |
| b) | Four masses m1, m2, m3 & m4 are 200 kg, 300 kg, 240 kg and 260 kg respectively. The corresponding radii of rotation are 0.2m, 0.15m, 0.25m and 0.3m respectively and the angles between successive masses are 45, 75 and 135. Find the position and magnitude of the balance mass required, if its radius of rotation is 0.2 m. (use graphical method). |  |
| c) | With the help of a neat sketch explain the construction and working of an Epiycydic Train Dynamometer. |  |