## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief annual maintenance and repair estimates. |  |
| b) | Explain the terms (i) Earnest money Deposit (ii) Security deposit. |  |
| c) | State the important characteristics of item rate contract. |  |
| d) | Bring out the importance and purpose of estimating. |  |
| e) | What is Measurement Book? Where it is used? State its importance. |  |
| f) | What are revised approvals? |  |
| g) | State the different methods of preparing approximate estimates for highways. |  |
| Q.No.2. | Answer any one of the following Questions: | (15) |
| a) | Fig Q.NO.2 shows plan and wall section of a single storeyed building. Calculate the quantities of any five items for the building: - Earthwork in excavation. - Masonry in foundation portion & plinth in CM 1:5 - External plastering 15mm thick with CM=1:3 - RCC roof slab 12 cm thick using concrete 1:11/2:3. - 12mm internal plaster 1:4 with neeru finish. - Flooring using ceramic tiles. - PCC 1: 3: 6 in foundation and plinth. |  |
| b) | Prepare an estimate for road portion 210 m long for the data given below. Use market rates for earthwork. Assume uniform falling gradient from chainage 1 to 8 of 1 in 200. Formation width of road is 12m. Keep side slope of 2:1 in banking and 1.5:1 in cutting. Assume final level to be maintained at chainage 1 to be 153.00m. Also draw longitudinal & cross section. Chainage; Distance in m; Existing RL of ground in metres. 1; 0; 154.25. 2; 30; 153.50. 3; 60; 153.75. 4; 90; 153.25. 5; 120; 152.85. 6; 150; 152.15. 7; 180; 150.75. 8; 210; 151.25. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Explain the various stages in execution of civil works by PWD. |  |
| b) | Explain the importance and types of records to be maintained at site. |  |

………2/-

-2-

| c) | Explain the term ‘Task Work’. What are the factors affecting task work? |  |
| --- | --- | --- |
| d) | Carry out rate analysis for RCC column 1:1.5:3 with 1.5% reinforcement, excluding shuttering and centering. |  |
| e) | Carry out rate analysis for brick masonry work in superstructure using 1:6 CM. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Explain in brief valuation and rent fixation of a Government building. |  |
| b) | What is a contract document? What are its important components? |  |
| c) | Explain the various stages involved in the tendering process. |  |
| d) | Draft a tender notice for inviting tenders for construction of a road at estimated cost of Rs.40 lakhs, to be completed in 5 months executed by PWD. |  |

| Q.No.5. | Write short notes on any three of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Classification of value |  |
| b) | Salient features of Indian Contract Act 1872 |  |
| c) | Importance of writing specifications |  |
| d) | Schedule of rates |  |