## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Architectural Engg.

Subject: Building Construction-II(4521)

# Time Duration: 3 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | In urban areas, where there is restriction of space, what type of structure will you design? Why? |  |
| b) | What is meant by ‘foundation’? Also state any two functions of foundation. |  |
| c) | Draw a cross section of a 300mm x 300mm brick pier and show the arrangement of bonds in ‘odd course’ and ‘even course’. |  |
| d) | What is the difference between an arch and a lintel? Sketch and explain. |  |
| e) | What type of paint do you recommend for: i)Internal painting ii)External painting iii)Steel work. |  |
| f) | Mention the type of stair to be constructed for: i) A residential building ii) Exhibition hall with different floors iii) Emergency exit stair for a hotel project. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the difference between a ‘Load bearing structure’ and a ‘framed structure’. |  |
| b) | Draw sketches of a ‘shallow foundation’ and ‘deep foundation’ and explain in brief. |  |
| c) | What are the classification of stone masonry? Explain the arrangement of stones in elevation and section. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the principles to be observed during construction of stone masonry? |  |
| b) | State the comparison between a stone masonry and brick masonry. |  |
| c) | What are the different types of bonds generally used in brick masonry? Explain the arrangement of bricks for a single brick wall in odd and even course with detail of mortar joints. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write down the points required to be considered during construction of brick masonry for a good quality work. |  |
| b) | Write short notes on: - Dressing of stone – Definition and purpose - Composite masonry and its types. |  |
| c) | If you are required to provide an RCC lintel for a window opening of 1.5 m, explain the construction procedure to be followed. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What is meant by an ‘Arch’, when and where it is provided? Also mention the classification with sketches. |  |
| b) | What type of flooring material will you recommend as an Architect for: i)An office building ii) Hospital building iii) A storage godown iv) Bath room v) Kitchen vi)Indoor stadium. |  |
| c) | Explain the procedure to be adopted while applying single coat cement plaster with neeru finish for an internal surface of a brick masonry wall to avoid any defects. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the general requirements to be considered for the construction of a good stair case. |  |
| b) | If a stair is to be provided surrounding a lift for a multi-storied building, show the arrangement of lift and stair in plan and section with suitable dimensions. |  |
| c) | State the classifications of stair case according to materials and explain the construction of any one in plan and section. |  |

***********