## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Drawing(4234)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of structural Engineers Handbook SP: 6(1) & IS 800

is permissible.

5) All Dimensions in the figures are in “mm” unless

specified.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write a note on the importance of position of arrow line in symbolic weld representation. |  |
| b) | Give the values of ‘Ixx’, ‘w’ and ‘a’ of the following steel rolled sections: i) ISBA 275 @ 45.6 Kg/m ii) ISLT 75 @ 7.1 Kg/m. |  |
| c) | Give the symbols of the following: i)Site HSFG Bolt ii) Open hole counter-sunk for side iii)Site Rivet counter-Sunk near side. |  |
| d) | Sketch a typical shoe joint (welded). |  |
| e) | Sketch a turn buckle pipe Hanger. |  |
| f) | Give three reasons for using pipe bends in pipelines. |  |
| g) | Determine the weight of the steel rolled sections given below. i) ISA 7550 x 6.0mm ii) ISA 15075 x 8.0mm |  |
| Q.No.2. | Draw the Fabrication Drawing of the Gusset Plates at “X” and “Y” and determine the exact length of the member X-Y, in the portion of the Truss, whose design drawing is shown in Fig.1. | (12) |
|  | NOTE FOR FIG.1: - All members are ISA 5050 x 5.0 mm - Gusset plate thickness is 6.0 mm iii) 12 mm  snap headed rivets are used. |  |

………2/-

-2-

| Q.No.3. | Answer the following: |  |
| --- | --- | --- |
| a) | Draw the Orthographic views and Developed view of the portion of the pipeline, whose pictorial view is shown in Fig.2. | (8) |
| b) | Explain the below mentioned terms used in Fabrication i) Cleat angles ii) Edge clearance iii) Gauge line iv) Bench mark v) Driving clearance vi) Bracing vii) Lacing viii)Bearing plate. ### OR | (4) |
| b) | Sketch a riveted plate girder and label its different components. | (4) |
| Q.No.4. | Answer the following: |  |
| a) | Draw the Top View and Front View of the support structure, whose pictorial line diagram is shown in fig 3. NOTE FOR FIG.3: All members used are ISA 5050 x 5.0 mm and all joints are welded joints, Butt or Fillet. No gusset plates are used at the joints. | (8) |
| b) | Make a bill of Material (BOM) of the support structure, shown in Fig.3, Q.4(a). ### OR | (4) |
| b) | Write a note on marketing and naming of different members in a structure for making a Bill of material (BOM). | (4) |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch a column splicing of bottom column ISHB 450 @ 87.2 Kg/m and top column ISMB @ 44.2 Kg/m. Indicate dimensions clearly. |  |
| b) | Interprete the symbolic weld representations shown in Fig 4. |  |
| c) | Sketch a double riveted, single strap, Zig-Zag, riveted joint, to join two plates of 10 mm and 12 mm together. Indicate dimension clearly. |  |
| Q.No.6. | Write notes on any two of the following: | 2 x 6 = 12 |
| a) | Different pressure pipe joints and their applications |  |
| b) | Need to convert a design drawing into fabrication drawing |  |
| c) | Importance of correct specifications of steel rolled sections |  |
|  |  | ….3/- |