## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Mechanical Engg./E/FT

Subject: Refrigeration & Airconditioning(5034)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define coefficient of performance for a refrigeration system. |  |
| b) | Designate the following refrigerants: - Trichloromonofluoro methane (CCl3F) - Ammonia. |  |
| c) | With a help of P-H diagram explain the effect of suction pressure on the performance of a simple vapour compression system. |  |
| d) | Define the following terms: - Specific humidity - Wet bulb depression. |  |
| e) | With a neat sketch briefly explain any Psychrometer. |  |
| f) | What are the requirements of good room air distribution? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between ‘Primary’ and ‘Secondary’ refrigerants. Enumerate the desirable properties of an ideal refrigerant. |  |
| b) | For ammonia based refrigerator working on vapour compression refrigeration system temperature range is given as –12C to 22C. If the gas is dry and saturated at the end of compression, calculate the C.O.P. of the cycle. Assume no under cooling. Temp; Enthalpy KJ/Kg; Entropy KJ/KgK. Liquid; Vapour; Liquid; Vapour. 22 C; 61.9; 208.1; 0.197; 0.703. -12 C; 18.4; 234.5; 0.075; 0.896. |  |
| c) | Explain with a neat sketch simple vapour absorption system. State how its efficiency can be improved. |  |
| Q.No.3. | Write short notes (with a neat sketch) on any three: | 3 x 4 = 12 |
| a) | Thermostatic expansion valve |  |
| b) | Water cooled condenser |  |
| c) | Flooded type evaporator |  |
| d) | Centrifugal compressor |  |
| e) | Plate type evaporator |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the construction and working of water cooler. |  |
| b) | 100m3 per minute of air at 15 dry bulb temperature and 60% RH is heated to a temperature of 30C. Find the amount of heat added to air per minute. |  |
| c) | Describe the following process with a neat sketch: - Cooling and dehumidification - Heating and dehumidification. | …..2/- |

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain with a neat sketch following duct systems: i) Loop perimeter duct system ii) Extended plenum duct system. |  |
| b) | What purpose is served by a fan? How are fans classified? Explain any one fan with a neat sketch. |  |
| c) | State the advantages of vapour compression refrigeration system over vapour absorption refrigeration system. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Grand sensible heat factor |  |
| b) | Time switch |  |
| c) | Automatic temperature control |  |
| d) | Duct design (any method) |  |
| e) | Application of central air conditioning plant. |  |

***********