## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Common

Subject: Elements of Mechanical Engineering(3004)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State advantages of V Belt drives. |  |
| b) | State the functions of Blow off cock. |  |
| c) | Name the different lubricants known to you and give one application of each. |  |
| d) | State Pascals Law and give its applications. |  |
| e) | What is Kinematic viscosity? State its units. |  |
| f) | Enumerate various losses of head in pipe. How contraction of pipe increases the loss of head? |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Find the power transmitted by a belt running over a pulley of 600 mm diameter at 200 rpm. The coefficient of friction between the belt and the pulley is 0.25, angle of lap 160 and the maximum tension in the belt is 2500 N. |  |
| b) | Draw a neat sketch of a compound gear train. Explain briefly the difference between simple and compound gear train. |  |
| c) | Discuss the working of a four stroke cycle diesel engine with the help of a neat sketch. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a package type boiler, label its parts and explain its working. |  |
| b) | Classify air compressors. Describe the working of a single stage reciprocating air compressor. |  |
| c) | Give the operational difficulties commonly experienced in centrifugal pumps and their remedies. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | State the desirable properties of a good lubricant. |  |
| b) | Differentiate between air cooling system and water cooling system. |  |
| c) | With the help of a neat sketch explain Forced lubrication system for an IC Engine. |  |
| Q.No.5. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Ball Bearing |  |
| b) | Economiser of a boiler |  |
| c) | Air motor |  |
| d) | Submersible pump |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Journal thrust bearing |  |
| b) | Use of compressed air |  |
| c) | Priming of centrifugal pump |  |
| d) | Ignition system |  |

***********