## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED/E&EE

Subject: Microprocessors(8085)-I(4142)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Interfacing. |  |
| b) | What do you understand by Peripheral Initiated operations? |  |
| c) | What do you mean by Nesting of subroutines. |  |
| d) | List the different addressing mode with an example for each. |  |
| e) | Define the terms: Instruction cycle and Machine cycle. |  |
| f) | Specify the register contents and flag status after the execution of the following instructions: MVI A, A9H MVI B, 57H ADD B ORA A HLT |  |
| h) | Give the difference between Maskable and Non-Maskable interrupts. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a neat diagram showing the Demultiplexing of the bus AD7 –AD0. |  |
| b) | Draw the flag register structure and explain the meaning of zero and carry flag. |  |
| c) | State the difference between: i)Accumulator and General Purpose Registers ii)Program Counter and Stack Pointer. |  |
| d) | Give the importance of Address Bus and Data Bus in IC 8085. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Give the meaning and indicate the number of bytes for the following instructions: - MVI C, 00H - CALL 8000H - SUB D - RLC |  |
| b) | Explain - Two 16 bit instructions - Machine Control Instructions. |  |
| c) | Draw the timing diagram showing the contents of the address and data buses and relevant control signals for the instruction XRI 89H. |  |
| d) | What do you mean by Direct and Indirect Addressing modes. Give examples for each. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A string of six data bytes is stored starting from memory location 2050H. The string includes some blanks (bytes with zero value). Write a program to eliminate the blanks from the strings and store the bytes in memory location starting from 3050H. |  |
| b) | Write a program to count from 00 to 20H with a delay of 100 msec between each count. After the count of 20H, the counter should reset itself and repeat the sequence. Use any register pair for the delay. |  |
| c) | Write a program to add five bytes of data stored in memory location starting from 3000H. Stop the addition if a carry is generated and store the sum in location 3050H. Otherwise continue adding and finally store the sum in location 3050H. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the PUSH and POP instructions. |  |
| b) | What do you understand by RIM and SIM instructions? |  |
| c) | Write note on RESTART Instruction. |  |
| d) | Explain concept of Subroutine with an example. |  |
| e) | Explain how the interfacing of any device (Input or output) to the Microprocessor takes place. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the 8085-interrupt process (INTR) in detail. |  |
| b) | Explain the steps involved in execution of CALL Instruction. |  |
| c) | Explain the Vectored Interrupts of 8085. |  |

***********