## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Substation Practice(5105)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare outdoor substation with indoor substation. |  |
| b) | State the advantages of sectionalization of bus. |  |
| c) | Give the general specifications for procurement of current transformer. |  |
| d) | State the advantages of earthing the neutral of a system. |  |
| e) | What is tarrif? Explain two part tarrif in detail. |  |
| f) | State the factors affecting current rating of a cable. |  |
| g) | Describe any two methods of charging of batteries. |  |
| h) | What are the different types of traction substations? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the classification of substations depending on the purpose. |  |
| b) | With the help of a neat diagram explain the double bus bar arrangement. What is the function of bus coupler? |  |
| c) | Draw a single line diagram of 33/11KV substation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Secondary circuit of current transformer should never be opened while primary is energized. Explain. |  |
| b) | With the help of neat diagram explain the working of different types of voltage transformers. |  |
| c) | Write a short note on relay and control panels for substation. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare isolated neutral and grounded neutral system. |  |
| b) | Explain any three lightening arrestors. |  |
| c) | State the causes of overvoltages. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the different methods of improving power factor of a system. |  |
| b) | A single phase motor connected to 240V, 50Hz supply takes 20A at a p.f. of 0.75 lagging. A capacitor is shunted across motor terminals to improve p.f. to 0.9. Determine capacitance of capacitor to be used. |  |
| c) | Give the general construction of cables with the help of a neat diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat diagram explain construction of oil filled cable. State the merits of the same. |  |
| b) | Write a short note on care and maintenance of lead acid batteries. |  |
| c) | Explain the use of silica gel. How is it reactivated? |  |
| d) | Explain the maintenance tests for insulating oil of the transformer. |  |

***********